*

* $Id: gtrak.doc,v 1.1.1.1 2002/07/24 15:56:26 rdm Exp $

*

* $Log: gtrak.doc,v $

* Revision 1.1.1.1  2002/07/24 15:56:26  rdm

* initial import into CVS

*

* Revision 1.1.1.1  2002/06/16 15:18:42  hristov

* Separate distribution  of Geant3

*

* Revision 1.1.1.1  1999/05/18 15:55:21  fca

* AliRoot sources

*

* Revision 1.1.1.1  1995/10/24 10:21:40  cernlib

* Geant

*

*

#include "geant321/pilot.h"

#if defined(CERNLIB_DOC)

*CMZ :  3.21/02 29/03/94  15.41.23  by  S.Giani

*-- Author :

*

************************************************************************

*                                                                      *

*                Introduction to the Tracking package                  *

*                ------------------------------------                  *

*                                                                      *

*                                                                      *

*  THE TRACKING PACKAGE                                                *

*                                                                      *

*  In  the context  of  simulation programs,  'tracking' a  particle   *

* through matter consists of predicting the spatial coordinates of a   *

* set of  points which  define the trajectory  and of  computing the   *

* components  of  the  momentum  at each  point.   This  is  usually   *

* achieved by integrating the  'equations of motion' over successive   *

* steps and applying  corrections when necessary to  account for the   *

* perturbations introduced by the interactions with matter.            *

*  The tracking package contains mainly a subprogram which controls,   *

* and effectively  performs, the transport  of all particles  in the   *

* current  event and  of  the secondary  products  which they  might   *

* possibly generate,  plus some  tools for  storing the  space point   *

* coordinates computed along the corresponding trajectories.           *

*                                                                      *

* THE STEP SIZE                                                        *

*                                                                      *

*  When tracking particles through a complex medium structure one of   *

* the critical tasks is the estimation  'a priori' of the step size.   *

* This is performed automatically by the program.                      *

*  For a particle with given  energy the step size depends primarily   *

* on the properties  of the particle (mass,  charge, lifetime, etc.)   *

* and of  the current medium in  which the particle is  moving.  The   *

* dependence may come either  from (quasi)continuous processes which   *

* usually  impose a  limit to  the interval  of integration  (energy   *

* loss, multiple scattering or bending  in magnetic field) or to the   *

* occurence of  a discrete process which  introduces a discontinuity   *

* in   the   trajectory    (decay,   electromagnetic   or   hadronic   *

* interaction).   In addition  to these  physical effects  there are   *

* constraints of a geometrical nature, the step being limited by the   *

* path length to the medium boundary.                                  *

*  In  practice,  the step  size  depends  ultimately  on a  set  of   *

* tolerances and cuts which should be  optimized by the user for the   *

* given application, such as:                                          *

*                                                                      *

* - the maximum bending angle due to magnetic field permitted in one   *

*   step,                                                              *

* - the maximum fractional energy loss in one step,                    *

* - the  maximum  step size  permitted  by  the multiple  scattering   *

*   theory used                                                        *

* - the accuracy for crossing medium boundaries and                    *

* - the absolute maximum step allowed                                  *

*                                                                      *

*  The limitations imposed  by the first three  processes could lead   *

* to extremely small steps for low energy particles.  To avoid this,   *

* in  GEANT has  been introduced  a minimum  step due  to continuous   *

* processes.   This represent  a lower  bound for  the maximum  step   *

* which is very important for keeping  the time taken to develop and   *

* follow a shower within reasonable limits.                            *

*                                                                      *

*  These  quantities are  part of  the so  called 'tracking  medium'   *

* parameters.  They  are usually calculated automatically  by GEANT,   *

* but may be provided by the user to be stored in the data structure   *

* JTMED, through the  routine GSTMED [CONS].  Usually,  this is done   *

* together with the initialisation  of the geometrical setup.  Users   *

* are  suggested   to  begin  their  simulations   with  the  values   *

* calculated by GEANT.   The optimisation is by no  means trivial as   *

* the economy of  computing time should not lead  to an unacceptable   *

* loss of accuracy.                                                    *

*  Other  general information  required for  the computation  of the   *

* step size is expected to be available in the data structures JPART   *

* and  JMATE,  for  the  properties  of the  particles  and  of  the   *

* materials,  and in  the  data structure  JVOLUM,  for the  current   *

* medium and its geometrical  boundaries.  The communication between   *

* the tracking package and the  structure JVOLUM is achieved through   *

* the  basic subroutines  of the  geometry package  GMEDIA (GTMEDI),   *

* GNEXT (GTNEXT) and GINVOL [GEOM,GTRAK]                               *

*  Some additional information is computed  at tracking time such as   *

* the probability  of occurence of an  interaction.  For convenience   *

* every particle is assigned a 'tracking  type', 1 for the gammas, 2   *

* for the  electrons and positrons,  3 for the neutral  hadrons (and   *

* neutrinos!), 4 for the charged hadrons and 5 for the muons.  Which   *

* physics processes  have potentially to  be considered for  a given   *

* particle depends on its tracking type.  For the hadrons it depends   *

* also, through  the subroutine GUPHAD, on  which hadronic processes   *

* have been selected (GHEISHA is the default) [PHYS 001].              *

*                                                                      *

* THE SUBROUTINES GTREVE and  GTRACK                                   *

*                                                                      *

*  At  event level  the  tracking is  controlled  by the  subroutine   *

* GTREVE called by  the subroutine GUTREV where the user  is free to   *

* take any other action.  GTREVE  loops over all vertices and stores   *

* all tracks  from the current vertex  in the stack JSTAK,  then for   *

* each  one  in  turn,  calls  GLTRAC to  prepare  the  commons  for   *

* tracking,  and starts  tracking through  a call  to GUTRAK,  which   *

* calls GTRACK.                                                        *

*  The subroutine GTRACK  tracks the particle up to the  end : stop,   *

* decay, interaction  or escape.   During this  phase it  may happen   *

* that secondary products  are generated and stored by  the user, as   *

* explained  below, in  the stack, and if  wanted, in  the permanent   *

* structure JKINE.                                                     *

*  The subroutine GTRACK loops over  all geometrical volumes seen by   *

* the  current  track,  first identifying,  through  the  subroutine   *

* GTMEDI, the new volume which  the particle has reached and storing   *

* the corresponding  material and  tracking medium constants  in the   *

* common blocks /GCMATE/ and /GCTMED/; the tracking is controlled by   *

* the  type-dependent  routines  GTELEC,GTGAMA,GTHADR,GTMUON,GTNEUT.   *

* These compute  the physical step  size according to  the activated   *

* physics processes, and compute the geometrical limit for the step,   *

* only when  necessary, through  GTNEXT, and propagate  the particle   *

* over the computed step.                                              *

*                                                                      *

* MAGNETIC FIELD ROUTINES                                              *

*                                                                      *

*  As mentioned before,  the effective propagation of the particles is *

* controlled by the routines GTGAMA, GTELEC, etc., which call GUSWIM.  *

* Depending on the  value chosen by the user for  the tracking medium  *

* parameter IFIELD the routine GUSWIM calls either                     *

*                                                                      *

* G3RKUTA    for inhomogeneous fields, IFIELD=1                         *

* G3HELIX    for quasi-homogeneous fields tilted w.r.t. the reference   *

*            frame, IFIELD=2, or                                        *

* G3HELX3    for one-component fields along the z axis, IFIELD=3        *

*                                                                      *

*  G3RKUTA and  G3HELIX call the  default user subroutine  GUFLD where   *

* the  components of  the field  at  the given  point are  computed.   *

* G3HELX3  takes  the value  of  the  field  in the  tracking  medium   *

* parameter FIELDM.                                                    *

*                                                                      *

* INFORMATION AVAILABLE AT TRACKING TIME, AND THE SUBROUTINE GUSTEP    *

*                                                                      *

*  At any  moment the  current track parameters  are available  in the *

* common block  /GCTRAK/ as well  as all  variables which have  to be  *

* preserved by  the tracking  routines for  the control  of the  step  *

* size.  In  addition a  few flags  and variables  are stored  in the  *

* common block /GCTRAK/ to record the history of the current step:     *

*                                                                      *

* INWVOL    is initialized  to 1 when  entering a new volume,  it is   *

*           set to  0 for all steps  inside the volume, to  2 if the   *

*           particle has reached  the volume boundary and  to 3 when   *

*           the particle is leaving the experimental setup.            *

* ISTOP     is initialized to 0 and set  to 1 if the particle looses   *

*           its identity or to 2 if it stops.                          *

*                                                                      *

*  The effect  which is responsible  for the limitation of  the step   *

* size as well as the corrective  effects which have been applied at   *

* the end of the step, if any,  are recorded in the first NMEC words   *

* of the mechanism vector LMEC and this is most useful to understand   *

* and debug the program.                                               *

*  The  total energy  loss for  the current  step is  stored in  the   *

* variable DESTEP, and the continuous energy loss in DESTEL.           *

*  This information  is necessary  for the user  to take  the proper   *

* actions in the subroutine GUSTEP which  is called by GTRACK at the   *

* end of every step.                                                   *

*  In addition, the  number NGKINE of secondary  products which have   *

* possibly been  generated and  their characteristics are  stored in   *

* the common  block /GCKING/ together with  an identification (array   *

* LMEC)  of   which  process  is  responsible.    Depending  on  the   *

* application  and on  the particle  type,  the user  may decide  in   *

* GUSTEP through the routine GSKING,                                   *

*                                                                      *

* - to discard the newly produced secondaries                          *

* - to  enter them  in  the  temporary JSTAK  data  structure to  be   *

*   tracked                                                            *

* - to  store them  in  the  permanent JKINE  data  structure to  be   *

*   tracked                                                            *

*                                                                      *

* CONNECTION WITH THE DETECTOR RESPONSE PACKAGE                        *

*                                                                      *

*  The loop over the volumes in  GTRACK makes the interface with the   *

* detector response  package simple [HITS].  By  construction of the   *

* geometrical setup  there is  a correspondance between  the volumes   *

* seen by  the particle and  the components of the  detectors.  When   *

* entering a new volume (in GTRACK) the subroutine GFINDS is called.   *

* If  the volume  has  been  declared by  the  user  as a  sensitive   *

* detector   through  appropriate   calls  to   GSDET  and   if  the   *

* corresponding tracking  medium constant ISVOL is  non zero, GFINDS   *

* returns in the  common block /GCSETS/ the  information to identify   *

* uniquely the detector component.  This  enables the user in GUSTEP   *

* to  record the  hits  properly  as well  as  the energy  deposited   *

* [HITS].                                                              *

*                                                                      *

* CONNECTION WITH THE DRAWING PACKAGE                                  *

*                                                                      *

*  The coordinates of the space points generated during the tracking   *

* are  available at  each step  in  the common  block /GCTRAK/.   In   *

* GUSTEP the user can store them in the structure JXYZ with the help   *

* of the subroutine  GSXYZ.  This information can be  used later for   *

* debug (subroutine  GPCXYZ) or for the  graphical representation of   *

* the trajectories [DRAW].                                             *

*                                                                      *

************************************************************************

#endif