*

* $Id: mat.doc 212 2007-10-04 23:45:50Z rdm $

*

* $Log: mat.doc,v $

* Revision 1.1.1.1  2002/06/16 15:18:35  hristov

* Separate distribution  of Geant3

*

* Revision 1.1.1.1  1999/05/18 15:55:15  fca

* AliRoot sources

*

* Revision 1.1.1.1  1995/10/24 10:19:51  cernlib

* Geant

*

*

*CMZ :  3.21/02 29/03/94  15.41.37  by  S.Giani

*-- Author :

C**

C** ALL MATERIAL CONSTANTS ARE PARTLY STORED ON THE COMMON /MAT IN THE

C** BLOCK DATA OR CALCULATED IN THE ROUTINE MATTER.IN PRESENT VERSION

C** THERE MAY BE DEFINED UP TO 20 MATERIALS WITH THE MATERIAL-INDEX

C** NUMBER 'IND' . THIS MATERIAL INDEX NUMBER MUST BE GIVEN FOR

C** EACH CELL OF A CALORIMETER IN THE GEOMETRY INPUT DATA CARDS.

C** (SEE COMMON /CALSTR AND INIT). IND =0 IS ALWAYS INTERPRETED AS

C** VACUUM IN THE PROGRAM.

C

C** DEN   (IND+1) =  DENSITY (GR/CM**3)

C** RADLTH(IND+1) =  RADIATION LENGTH (CM)

C** ATNO  (IND+1) =  ATOMIC NUMBER (AVERAGE VALUE, SEE LATER)

C** ZNO   (IND+1) =  CHARGE NUMBER OF NUCLEUS (AVERAGE VALUE, SEE LATER)

C** ABSL  (IND+1) =  ABSORPTION LENGTH (ONLY USED FOR STEP SIZE

C**                  CALCULATION)

C** CDEN  (IND+1) =  C - PARAMETER IN STERNHEIMERS THEORY

C** MDEN  (IND+1) =  M - PARAMETER IN STERNHEIMERS THEORY

C** X0DEN (IND+1) =  X0- PARAMETER IN STERNHEIMERS THEORY

C** X1DEN (IND+1) =  X1- PARAMETER IN STERNHEIMERS THEORY

C** RION  (IND+1) =  IONISATION POTENTIAL

C** MATID (IND+1)    =  GHEISHA ALPHANUMERIC LABEL (A4)

C** MATID1(IND+1,24) =  EGS ALPHANUMERIC LABEL (24*A1)

C**   10 PARAMETERS DESCRIBING THE MATERIAL AT VARIOUS PLACES

C**   IN THE PROGRAM :

C** PARMAT(IND+1, 1) = X    DE/DX CORRECTION FACTOR FOR LANDAU

C**                         TYPE DISTRIBUTIONS

C** PARMAT(IND+1, 2) = 1.   FOR SOLIDS

C**                  = 2.   FOR FLUIDS

C**                  = 3.   FOE GASEOUS MATERIALS

C** PARMAT(IND+1, 3) = 1.   DELTA-RAY SELECTION : ALL PARTICLES

C**                  = 2.   DELTA-RAY SELECTION : ONLY MUONS

C**                  = 3.   DELTA-RAY SELECTION : MUONS AND ELECTRONS

C**                  = 4.   DELTA-RAY SELECTION : MUONS,ELECTRONS,PIONS

C** PARMAT(IND+1, 4) =      MINIMUM RANGE FOR DELTA-ELECTRONS

C** PARMAT(IND+1, 5) = 1.   FOR ANORGANIC SCINTILLATORS ONLY

C** PARMAT(IND+1, 6) =      SATURATION COEFFICIENT IN BIRK'S LAW

C**                  = 1.   DEFAULT VALUE  SAT=0.010/DENSITY,

C**                  > 1.                  SAT=PARMAT(IND+1,6)-1.

C** PARMAT(IND+1, 7) > 0.   INDUCED HADRON NUCLEAR FISSION

C** PARMAT(IND+1, 8) > 0.  PHOTOABSORBTION AND PHOTOFISSION

C** PARMAT(IND+1, 9) > 0.  NATURAL RADIOACTIVITY

C** PARMAT(IND+1,10) =     CORRECTION FACTOR FOR ENERGY LOSS BY

C**                        NUCLEAR BREAKUP EFFECTS

C**                  = 0.  DEFAULT MODEL DESCRIPTION IS USED.

C**

C**    THE DENSITY, ATOMIC NUMBER AND CHARGE NUMBER , DEFINED ABOVE,

C**    ARE AVERAGE VALUES FOR COMPOUND MATERIALS LIKE BGO.

C**    THESE VALUES ARE USED FOR DE/DX- CALCULATIONS ONLY.

C**

C** ALL MATERIALS MAY BE DEFINED AS COMPOUNDS OR MIXTURES,

C** THE FOLLOWING NUMBERS ARE TAKEN FROM THE EGS DATA FILE,

C** OR, IF NOT GIVEN, CALCULATED.

C** IFRAC(J)      = NUMBER OF COMPOUNDS IN THIS MATERIAL

C** FRAC1(J,I)    = PARTIAL INTERACT.LENGTH OF THE I.TH COMPOUND

C** DEN1 (J,I)    = PARTIAL DENSITY   OF THE I.TH COMPOUND

C** ATNO1(J,I)    = ATOMIC NUMBER     OF THE I.TH COMPOUND

C** ZNO1 (J,I)    = CHARGE NUMBER     OF THE I.TH COMPOUND

C**

C**