*

* $Id: gdraw.doc 212 2007-10-04 23:45:50Z rdm $

*

* $Log: gdraw.doc,v $

* Revision 1.1.1.1  2002/06/16 15:18:38  hristov

* Separate distribution  of Geant3

*

* Revision 1.1.1.1  1999/05/18 15:55:03  fca

* AliRoot sources

*

* Revision 1.1.1.1  1995/10/24 10:20:18  cernlib

* Geant

*

*

#include "geant321/pilot.h"

#if defined(CERNLIB_DOC)

*CMZ :  3.21/02 29/03/94  15.41.25  by  S.Giani

*-- Author :

*

************************************************************************

*                                                                      *

*             Introduction to the Drawing package                      *

*             -----------------------------------                      *

*                                                                      *

*                                                                      *

* THE DRAWING PACKAGE                                                  *

*                                                                      *

* The drawing package has been designed mainly to:                     *

*                                                                      *

* - draw the detector                                                  *

* - draw the detector geometrical tree                                 *

* - draw particle trajectories                                         *

* - draw hits.                                                         *

*                                                                      *

*  DRAWING THE DETECTOR                                                *

*                                                                      *

*  The detector  can be looked at  from any view point  and with any   *

* scale factor (GDRAW, GDRVOL); appropriate attributes can be set in   *

* order to see  only selected objects (so  avoiding messy pictures);   *

* hidden line removal is available as well as surface shading.         *

*  'Cut'  views,  i.e.  intersections  of a  given  plane  with  the   *

* detector, can also be displayed (GDRAWC, GDRAWX).  This feature is   *

* very useful to  see internal details that the  routine GDRAW would   *

* not show.                                                            *

*  When hidden line  removal is used, the possibility  exists to cut   *

* the volume to be drawn with various shapes, to visualise the inner   *

* details.                                                             *

*                                                                      *

* DRAWING THE GEOMETRICAL TREE                                         *

*                                                                      *

*  The geometrical tree (GDTREE) is  a representation of the overall   *

* structure of the detector, namely the mother-daughter relationship   *

* among  the  various  objects   composing  it.   Several  types  of   *

* additional information are available on request: multiplicity of a   *

* given  volume (i.e.  how  many  times it  is  positioned in  other   *

* places, or  number of  subdivisions), detector  nature, visibility   *

* flag, etc.  This  drawing tree capability can be  very useful when   *

* tuning  the  detector  geometry.    If  used  interactively,  this   *

* facility  allow to  invoke via  a click  of the  mouse the  GDSPEC   *

* utility (see below) for any node of the tree.                        *

*                                                                      *

* DRAWING THE GEOMETRICAL SPECIFICATIONS                               *

*                                                                      *

*  The geometrical  specifications (GDSPEC) give a  detailed picture   *

* of one  particular piece of  the detector.  Three drawings  of the   *

* volume (a projection  view and two cut views), its  shape type and   *

* numerical  parameters  (i.e.  dimensions),  and  a  scale  to  aid   *

* geometric  calculations,  are  presented   together  in  a  single   *

* graphics frame.  The set of  geometrical specifications of all the   *

* descendants of  a given node on  the tree, can easily  be obtained   *

* with the routine GDFSPC.                                             *

*                                                                      *

* DRAWING PARTICLE TRAJECTORIES                                        *

*                                                                      *

*  The tracks generated  by the tracking package, and  stored in the   *

* data structure JXYZ, can be easily drawn with the routine GDXYZ.     *

*  The names of the particles and/or  the track numbers can be drawn   *

* as well (GDPART).                                                    *

*  Four types of representations are  used to display the classes of   *

* particles, with different colour and line style:                     *

*                                                                      *

* - red solid lines for charged particles (GTELEC, GTHADR)             *

* - green dashed lines for muons (GTMUON)                              *

* - black blank/dotted lines for neutral particles (GTNEUT)            *

* - blue dotted lines for gammas (GTGAMA)                              *

*                                                                      *

*  A special routine has been  provided to display the tracks online   *

* (GDCXYZ), if  called under  the DEBUG/SWITCH control  from GUSTEP.   *

* That routine  shows the tracks exactly  at the same time  they are   *

* trasorted by  the tracking package  of GEANT3, giving so  a useful   *

* interactive debugging tool.                                          *

*                                                                      *

* DRAWING HITS                                                         *

*                                                                      *

*  The hits generated by the tracking package and stored in the        *

* data structure JHITS, can be displayed by the hits routines,         *

* with different functionality:                                        *

*                                                                      *

* - draw one hit (GDAHIT); called by user routines                     *

* - draw all the hits of trajectory type sets/detectors (GDHITS)       *

* - draw all the hits of calorimeter type sets/detectors (GDCHIT)      *

*                                                                      *

*  Different symbols for every subdetector can be used, chosen among   *

* hardware characters  (dots), software  crosses, or from  the HPLOT   *

* table of software characters.  The size of the software characters   *

* and crosses is given as an  argument to GDAHIT/GDHITS, while it is   *

* computed as a function of the hits value in GDCHIT.                  *

*                                                                      *

* THE VIEW BANKS                                                       *

*                                                                      *

*  The basic detector drawing  routines (GDRAW, GDRAWC, GDRAWX) have   *

* to scan the  data structure JVOLUM repeatedly.   When the detector   *

* is  described in  a  very  detailed way,  the  time  spent in  the   *

* interpretation of  the JVOLUM bank  and in the  3D transformations   *

* can  increase dramatically.   For a  detector with  more than  100   *

* different volume names, for example, this time can reach real time   *

* minutes on some machines, whereas  the specific time required just   *

* by the  drawing would be only  a few seconds.  If  the hidden line   *

* removal option is active, this  time can reach several minutes and   *

* even  hours,  depending  on  the  speed of  the  machine  and  the   *

* complexity of the drawing requested.                                 *

*  In order to alleviate this  problem the 'bank-mode' routines have   *

* been developed.  The basic idea  is to separate the interpretation   *

* (i.e. the  JVOLUM scanning  to convert  the 3D  geometry structure   *

* into a set of 2D lines) from the drawing itself.  In this way, the   *

* interpretation is performed  only once and all  the 2D information   *

* is stored in view banks  (data structure JDRAW) [DRAW 399].  These   *

* views can then be looked at in  a quicker way, having only to draw   *

* all 2D vectors  previously stored.  For a detector  with more than   *

* 100 different volume  names, for example, this is  achieved at the   *

* cost of only a few thousand words of memory for each view bank.      *

*  One  can therefore  open a  view bank  (GDOPEN), identified  by a   *

* number, perform appropriate drawings  (only interpretation will be   *

* made, of course), close the bank  (GDCLOS) and finally look at the   *

* picture stored in  it (GDSHOW).  When a view bank  has been closed   *

* it cannot  be modified anymore,  but it  can be displayed  as many   *

* times as wanted (GDSHOW) or deleted (GDELET).                        *

*                                                                      *

* OTHER FEATURES                                                       *

*                                                                      *

*  The user can  control some drawing options  (GDOPT), by selecting   *

* for instance  to have  either parallel or  perspective projection,   *

* either  Y-Z  or  R-Z  projection,  hidden  line  removal,  surface   *

* shading, etc.                                                        *

*  There is  a routine (GDZOOM) that,  if called, applies a  zoom to   *

* everything (volumes, tracks, hits, etc.)  will be drawn from then.   *

* This feature, in conjunction  with the interactive command MEASURE   *

* [XINT 110], can be used for detailed viewing.                        *

*  Another  tool that  could help  in the  interactive debugging  or   *

* tuning of the detector geometry  is the routine GEDITV [DRAW 600],   *

* by which it  is possible to modify  interactively some geometrical   *

* parameters  set  by  the   user  routines  defining  the  detector   *

* geometry.                                                            *

*  It is possible to draw the axes of the 3D MAster Reference System   *

* (GDAXIS) oriented in agreement with the current view point.          *

*  Two other  routines draw a scale  (GDSCAL) or a profile  of a man   *

* (GDMAN) in 2D  user coordinates to give an idea  of the dimensions   *

* within current scale  factors.  A 2D text  (GDRAWT) using software   *

* characters (hardware  characters should be implemented  later on),   *

* 2D vectors (GDRAWV) or a frame header (GDHEAD) are also available.   *

*  Attributes  like colour  (GDCOL)  and line  width  (GDLW) can  be   *

* globally set for all 2D  drawings (i.e. text, vectors, man, etc.);   *

* they are overridden in 3D drawings by volume attributes set by the   *

* GSATT routine with 'COLO' or 'LWID' option.                          *

*  A  graphics input  is available  (GDCURS)  to fetch  the 2D  user   *

* coordinates  of the  graphics cursor  on the  screen, allowing  an   *

* immediate user  interface with the interactive  version of GEANT3.   *

* In  particular there  are interactive  commands to  zoom, measure,   *

* pick tracks or hits points that make use of that routine.            *

*  Various conversions from  3D to 3D, and 3D to  2D coordinates are   *

* performed by GDFR3D and GD3D3D.                                      *

*                                                                      *

* BASIC AND ADVANCED GRAPHICS                                          *

*                                                                      *

*  The  underlying graphics  system  is completely  hidden from  the   *

* GEANT program.  All  graphics call are made via  the HIGZ package.   *

* Various implementations of the HIGZ package are available, notably   *

* for X11, GKS, DI3000 and, shortly, PHIGS.                            *

*                                                                      *

*  RUNNING INSTRUCTIONS                                                *

*                                                                      *

*  Thanks to the HIGZ package,  it is possible to produce postscript   *

* metafiles from the drawings.  When  the GKS implementation of HIGZ   *

* is used, the possibility is there  to produce also a GKS metafile.   *

*                                                                      *

* SUMMARY                                                              *

*                                                                      *

*  The drawing package is initialized by (in the order):               *

*  IGINIT    to initialize the HIGZ package and the underlying basic   *

*            graphics software                                         *

*  GDINIT    to initialize the GEANT drawing package                   *

*                                                                      *

*  Main drawing routines are:                                          *

*  GDRAW     to draw a projection view of the detector                 *

*  GDRVOL    to draw a projection view of the detector                 *

*  GDRAWC    to draw a cut view of the detector (along one axis)       *

*  GDRAWX    to draw a cut view of the detector (from any point)       *

*  GDXYZ     to draw tracks at the end of the event                    *

*  GDCXYZ    to draw tracks while the event is running                 *

*  GDPART    to draw particle names and track numbers on tracks        *

*  GDAHIT    to draw one single hit                                    *

*  GDHITS    to draw hits for trajectory type detectors                *

*  GDCHIT    to draw hits for calorimeter type detectors               *

*                                                                      *

*  Routines that show how the detector has been modeled are:           *

*  GDTREE    to draw a picture with the geometrical tree               *

*  GDSPEC    to draw a picture with volume specifications              *

*  GDFSPC    to draw several GDSPEC pictures                           *

*                                                                      *

*  Routines that perform control operations on view banks are:         *

*  GDOPEN    to open a given view bank, identified by a view number;   *

*            in this way we enter in bank-mode                         *

*  GDCLOS    to close  current view bank, i.e. the last  one opened,   *

*            and restore screen-mode                                   *

*  GDSHOW    to show all graphics information contained  in a  given   *

*            view bank                                                 *

*  GDELET    to delete a given view bank from memory                   *

*                                                                      *

*  Other routines are:                                                 *

*  GDOPT     to set drawing options                                    *

*  GDZOOM    to set the zoom parameters                                *

*  GDAXIS    to draw the axes of  the MARS,  oriented according        *

*            to the current view parameters                            *

*  GDSCAL    to draw the current scale                                 *

*  GDMAN     to draw a profile of a man  (or a woman) at the current   *

*            scale                                                     *

*  GDRAWT    to draw text, with software characters                    *

*  GDRAWV    to draw polylines in 2D user coordinates                  *

*  GDHEAD    to draw a frame header                                    *

*  GDCOL     to set colour code                                        *

*  GDLW      to set line width                                         *

*  GDCURS    to have an input from the graphics cursor                 *

*  GDFR3D    to convert from 3D coordinates (either in MARS or DRS)    *

*            to 2D user coordinates                                    *

*  GD3D3D    to convert from 3D MARS coordinates to  3D  Projection    *

*            Reference System coordinates.                             *

*                                                                      *

*        Labelled COMMON blocks related to section DRAW                *

*        ----------------------------------------------                *

*                                                                      *

*    COMMON/GCDRAW/NUMNOD,MAXNOD,NUMND1,LEVVER,LEVHOR,MAXV,IPICK,      *

*   + MLEVV,MLEVH,NWCUT,JNAM,JMOT,JXON,JBRO,JDUP,JSCA,JDVM,JPSM,       *

*   + JNAM1,JMOT1,JXON1,JBRO1,JDUP1,JSCA1,JULEV,JVLEV,                 *

*   + LOOKTB(16),                                                      *

*   + GRMAT0(10),GTRAN0(3),IDRNUM,GSIN(41),GCOS(41),SINPSI,COSPSI,     *

*   + GTHETA,GPHI,GPSI,GU0,GV0,GSCU,GSCV,NGVIEW,                       *

*   + ICUTFL,ICUT,CTHETA,CPHI,DCUT,NSURF,ISURF,                        *

*   + GZUA,GZVA,GZUB,GZVB,GZUC,GZVC,PLTRNX,PLTRNY,                     *

*   + LINATT,LINATP,ITXATT,ITHRZ,IPRJ,DPERS,ITR3D,IPKHIT,IOBJ,LINBUF,  *

*   + MAXGU,MORGU,MAXGS,MORGS,MAXTU,MORTU,MAXTS,MORTS,                 *

*   + IGU,IGS,ITU,ITS,NKVIEW,IDVIEW,                                   *

*   + NOPEN,IGMR,IPIONS,ITRKOP,IHIDEN,                                 *

*   + DDUMMY(18)                                                       *

* C                                                                    *

* NUMNOD  number of nodes in non-optimized tree                        *

* MAXNOD  max. number of nodes of non-optimized tree.                  *

* NUMND1  number of nodes in optimized tree                            *

* LEVVER  vertical level in the tree currently scanned                 *

* LEVHOR  horizontal node in the tree currently scanned                *

* MAXV    max vertical levels in the tree to be scanned                *

* IPICK   node selected by GDTREE                                      *

* MLEVV   number of vertical levels in the last tree scanned           *

* MLEVH   number of horizontal nodes in the last tree scanned          *

* NWCUT   max. workspace allocated by cut routines                     *

* JNAM-JVLEV   pointers used by the tree routines                      *

* LOOKTB  colour look-up table, LOOKTB(I)=I,I=1,16                     *

* GRMAT0  rotation matrix saved by GDRVOL                              *

* GTRAN0  translation matrix saved by GDRVOL                           *

* IDRNUM  flag for GDRAW, set to 1 when called by GDRVOL               *

* GSIN    sine table                                                   *

* GCOS    cosine table                                                 *

* SINPSI  SIN(GPSI*DEGRAD)                                             *

* COSPSI  COS(GPSI DEGRAD)                                             *

* GTHETA  Theta angle of the parallel projection of 3-D images         *

* GPHI    Phi angle of the parallel projection of 3-D images           *

* GPSI    Psi angle of rotation of the image on the screen             *

* GU0     U position (X in screen coordinates) of the origin           *

* GV0     V position (Y in screen coordinates) of the origin           *

* GSCU    scale factor for the U screen coordinate                     *

* GSCV    scale factor for the V screen coordinate                     *

* NGVIEW  flag for GDFR3D and GD3D3D if view point has changed         *

* ICUTFL  flag for GDRAW if it was called by cut routines              *

* ICUT    axis along which the cut is performed                        *

* CTHETA  Theta angle of cut supplied to GDRAWX                        *

* CPHI    Phi angle of cut supplied to GDRAWX                          *

* DCUT    coordinate value at which the cut is performed               *

* NSURF   number of surfaces stored in SURF                            *

* ISURF   pointer for array SURF                                       *

* GZUA    zoom parameter (horizontal scale factor)                     *

* GZVA    zoom parameter (vertical scale factor)                       *

* GZUB    zoom parameter                                               *

* GZVB    zoom parameter                                               *

* GZUC    zoom parameter                                               *

* GZVC    zoom parameter                                               *

* PLTRNX  screen and plotter X range                                   *

* PLTRNY  screen and plotter Y range                                   *

* LINATT  current line attributes                                      *

* LINATP  permanent line attributes                                    *

* ITXATT  current text attributes                                      *

* ITHRZ   string containing the status of THRZ option                  *

* IPRJ    string containing the status of PROJ option                  *

* DPERS   distance from the origin for perspective view                *

* ITR3D   track being scanned                                          *

* IPKHIT  flag for GPHITS, if>0 then print only hit number             *

* IOBJ    type of the object being drawn                               *

* LINBUF  flag for GDRAWV if line buffering is active                  *

* MAXGU   current physical number of words for graphic unit banks      *

* MORGU   number of words to be pushed in graphic unit banks           *

* MAXGS   current physical number of words for graphic segment banks   *

* MORGS   number of words to be pushed in graphic segment banks        *

* MAXTU   current physical number of words for text unit banks         *

* MORTU   number of words to be pushed in text unit banks              *

* MAXTS   current physical number of words for text segment banks      *

* MORTS   number of words to be pushed in text segment banks           *

* IGU     pointer to current graphic unit bank                         *

* IGS     pointer to current graphic segment bank                      *

* ITU     pointer to current text unit bank                            *

* ITS     pointer to current text segment bank                         *

* NKVIEW  number of view data banks                                    *

* IGVIEW  current view bank number or 0 for screen                     *

* NOPEN   unused                                                       *

* IGMR    unused                                                       *

* IPIONS  unused                                                       *

* ITRKOP  status of TRAK option of GDOPT                               *

* DDUMMY  array of dummy words                                         *

*                                                                      *

*                                                                      *

*                                                                      *

*                The View data structure JDRAW                         *

*                -----------------------------                         *

*                                                                      *

* NKVIEW    Number of views                                            *

* IVIEW     Current view selected                                      *

* IGU       Current graphic unit pointer                               *

* MAXGU     Number of graphic units                                    *

* MORGU     Number of words to push in graphic unit bank               *

* IGS       Current graphic segment pointer                            *

* MAXGS     Number of graphic segments                                 *

* MORGS     Number of words to push in graphic segment bank            *

* ITU       Current text unit pointer                                  *

* MAXTU     Number of text units                                       *

* MORTU     Number of words to push in text unit bank                  *

* ITS       Current text segment pointer                               *

* MAXTS     Number of text segments                                    *

* MORTS     Number of words to push in text segment bank               *

* LENGU     Array containing: lengths for each graphic unit + LINATT   *

*           (line attributes)                                          *

* ADDGU     Array containing addresses for each graphic unit           *

* ADDTU     Array containing addresses for each text unit              *

* X         Array containing u-coordinates of graphic segments         *

* Y         Array containing v-coordinates of graphic segments         *

* ICUT      Cut axis (1,2,3 or 0 if no cut) of the view                *

* LINWID    Text line width + ITXATT (text attributes)                 *

* GTHETA,  GPHI,  GPSI,  GU0,  GV0,  GSCU,  GSCV,  are  the  viewing   *

* parameters stored in /GCDRAW/.                                       *

* U0, V0,  SIZE, ANGLE, IOPT, ITEXT  have the same meaning  of those   *

* given as parameters in HPLSOF or GDRAWT routines.                    *

*  A control word is stored  in Q(JDRAW+IVIEW), to identify the view   *

* banks among three classes:                                           *

*                                                                      *

*  =1 for empty  banks (created just  to avoid gaps) or  for deleted   *

*     banks;                                                           *

*  =2 for all previously created banks (i.e. opened);                  *

*  =3 for protected  banks (all banks  that can't be deleted  by the   *

*     user with GDELET, because reserved for internal use).            *

*                                                                      *

*                           | JDRAW                                    *

*                           |                                          *

*     NKVIEW   IVIEW        v                 NKVIEW                   *

*       ........................................                       *

*       |     | |          | |  Control words  |                       *

*       ........................................                       *

*              |                                                       *

*              | JV = LQ(JDRAW-IVIEW)                                  *

*  6           v                                           22          *

* ............................................................         *

* | | | | | | | |igu,maxgu,morgu,igs,maxgs,morgs,itu,maxtu,  |         *

* | | | | | | | |        mortu,its,maxts,morts,gtheta,gphi,  |         *

* | | | | | | | |        gpsi,gu0,gv0,gscu,gscv,-,-,icut     |         *

* ............................................................         *

*  | | | | | |                                                         *

*  | | | | | | JV1 = LQ(JV-1)                                          *

*  | | | | | v                                          MAXGU          *

*  | | | | |..................................................         *

*  | | | | || | lengu(1) | .... | lengu(igu) | .... | lengu(mgu)       *

*  | | | | |..................................................         *

*  | | | | |                                                           *

*  | | | | | JV2 = LQ(JV-2)                                            *

*  | | | | v                                            MAXGU          *

*  | | | |....................................................         *

*  | | | || | addgu(1) | ..... | addgu(igu) | ..... | addgu(mgu)       *

*  | | | |....................................................         *

*  | | | |                                                             *

*  | | | |  JV3 = LQ(JV-3)                                             *

*  | | | v                                              MAXTU          *

*  | | |......................................................         *

*  | | || | addtu(1) | ...... | addtu(itu) | ...... | addtu(mtu)       *

*  | | |......................................................         *

*  | | |                                                               *

*  | | | JV4 = LQ(JV-4)                                                *

*  | | v                                                MAXGS          *

*  | |........................................................         *

*  | || |  x(1)  |  ......  |  x(igs)  |  ......  |  x(mgs)  |         *

*  | |........................................................         *

*  | |                                                                 *

*  | | JV5 = LQ(JV-5)                                                  *

*  | v                                                  MAXGS          *

*  |..........................................................         *

*  || |   y(1)  |  ......  |  y(igs)  |  ......  |  y(mgs)   |         *

*  |..........................................................         *

*  |                                                                   *

*  | JV6 = LQ(JV-6)                                                    *

*  v                                                    MAXTS          *

* ............................................................         *

* | |u0(1)|v0(1)|size(1)|angle(1)|linwid(1)|iopt(1)|itext(1)|nchar(1)|.*

*  ............................................................        *

*                                                                      *

************************************************************************

#endif