*

* $Id: gphys.doc,v 1.1.1.1 2002/07/24 15:56:25 rdm Exp $

*

* $Log: gphys.doc,v $

* Revision 1.1.1.1  2002/07/24 15:56:25  rdm

* initial import into CVS

*

* Revision 1.1.1.1  2002/06/16 15:18:41  hristov

* Separate distribution  of Geant3

*

* Revision 1.1.1.1  1999/05/18 15:55:20  fca

* AliRoot sources

*

* Revision 1.1.1.1  1995/10/24 10:21:19  cernlib

* Geant

*

*

#include "geant321/pilot.h"

#if defined(CERNLIB_DOC)

*CMZ :  3.21/02 29/03/94  15.41.21  by  S.Giani

*-- Author :

*

************************************************************************

*                                                                      *

*                  Introduction to the section PHYS                    *

*                  --------------------------------                    *

*                                                                      *

*                                                                      *

* THE PHYSICS PROCESSES                                                *

*                                                                      *

*  The processes  currently implemented in GEANT3  can be classified   *

* as follows:                                                          *

*                                                                      *

* - Decays in flight                                                   *

* - Multiple scattering (Gaussian or Moliere)                          *

* - Continuous electromagnetic processes                               *

* - Discrete electromagnetic processes                                 *

* - Hadronic interactions                                              *

* - Muon nucleus-interactions.                                         *

*                                                                      *

*  The  fist  two  are  controlled  by  the  routines  GDECAY  which   *

* generates the decay products and  GMULTS which computes the change   *

* in  the angle  due to  multiple scattering  over a  given tracking   *

* step.   The  others  are  reviewed  separately  in  the  following   *

* paragraphs.                                                          *

*  For  convenience the  particles are  given a  'tracking type'  in   *

* GEANT, depending on their interaction with matter:                   *

*                                                                      *

*   ITRTYP = 1  photon                                                 *

*            2  electron and positron                                  *

*            3  neutral particles                                      *

*            4  charged hadrons                                        *

*            5  muon                                                   *

*            6  'Geantino'                                             *

*            7  Cerenkov photon                                        *

*            8  Ion                                                    *

*                                                                      *

*  The  paragraph  5  gives  a  summary  of  the  physics  processes   *

* activated for  each type  of particle with  the momentum  range of   *

* validity when relevant.  GPHYSI controls the initialisation of the   *

* various processes.                                                   *

*                                                                      *

* CONTINUOUS ELECTROMAGNETIC PROCESSES                                 *

*                                                                      *

* GMOLI     Initializes Moliere scattering                             *

* GPROBI    initializes material 'constants'  used for computing the   *

*           probability of various interactions.                       *

* GDRELA    Control routine  to fill   DE/DX tables for  energy loss   *

*           due to ionisation.                                         *

* GDRELP    Calculates  energy loss  due to  ionisation for  charged   *

*           particles other than electrons and positrons.              *

* GDRELE    Calculates energy  loss due to ionisation  for electrons   *

*           and  positrons.  As  Moller  and  Bhabba scattering  are   *

*           treated  as discrete  processes the  energy loss  due to   *

*           ionisation is a function  of the electron kinetic energy   *

*           cut-off DCUTE below which these processes are treated as   *

*           a continuous energy loss [BASE 030, common /GCPHYS/].      *

* GBRELA    Initializes   Bremsstrahlung  cross-section   and  fills   *

*           energy  loss tables  for Bremsstrahlung.   Both formulae   *

*           depend on  the photon  energy cut-off BCUTE  below which   *

*           Bremsstrahlung is  treated as  a continuous  energy loss   *

*           [BASE 030, common /GCPHYS/].                               *

* GPRELA    Fills  DE/DX  tables  for  energy loss  by  direct  pair   *

*           production  for high  energy  muons.  The  corresponding   *

*           discrete process is not considered.                        *

* GRANGI    Calculates    the    stopping     range    tables    for   *

*           electron/positron, muon and proton.                        *

* GMULOF    Calculates the tables for the maximum step allowed to to   *

*           continuous processes: Bending  in magnetic field, energy   *

*           loss and multiple scattering.                              *

* GCOEFF    Calculates  the   interpolation  coefficients   for  the   *

*           energy/Range relation  to be used in  the calculation of   *

*           the energy loss.                                           *

*                                                                      *

* DISCRETE ELECTROMAGNETIC PROCESSES                                   *

*                                                                      *

*   For the simulation of each  given discrete physics process three   *

* tasks have to be performed:                                          *

*                                                                      *

* - The evaluation  of the step  length.  This is computed  from the   *

*   updated  probability for  the  occurrence of  the process.   The   *

*   corresponding code  has been  inserted directly in  the tracking   *

*   routines for each tracking type as appropriate.                    *

* - After transport  of the  particle, the  generation of  the final   *

*   state particles (GEANT routines GPHOT, GCOMP, etc.)                *

* - If the  particle survives  after the interaction,  recompute the   *

*   probability.                                                       *

*                                                                      *

*   It should be outlined that the  evaluation of the step length is   *

* made independently  for each  process which  can occur,  the final   *

* step  size  being the  minimum  of  all  and the  process  finally   *

* considered being the corresponding one.                              *

*                                                                      *

* GPHOTI    Tabulates  cross-section  for photo-electric  effect  at   *

*           initialisation time                                        *

* GPHOT     Simulates photo-electric mechanism                         *

* GCOMPI    Tabulates  cross-section  for  Compton  intersection  at   *

*           initialisation time                                        *

* GCOMP     Simulates Compton scattering                               *

* GPRSGA    Tabulates   cross-section   for   pair   production   at   *

*           initialisation time                                        *

* GPAIRG    Simulates electron pair production by photons              *

* GBRSGA    Tabulates    cross-section    for   Bremsstrahlung    at   *

*           initialisation time                                        *

* GBREME    Simulates  hard  Bremsstrahlung   by  electrons.  -  see   *

*           cut-off BCUTE in routine GBRELA above                      *

* GDRSGA    Tabulates cross-section for  delta-ray at initialisation   *

*           time                                                       *

* GDRAY     Simulates delta rays (Moller or Bhabba scattering) - see   *

*           cut-off DCUTE in routine GDRELE above                      *

* GANNII    Tabulates  cross-section  for positron  annihilation  at   *

*           initialisation time                                        *

* GANNI     Simulates positron annihilation in flight                  *

* GANNIR    Simulates positron annihilation at rest.                   *

* GPFISI    Tabulates cross-section for photo-fission                  *

* GPFIS     Simulates photo-fission                                    *

*                                                                      *

* HADRONIC PROCESSES                                                   *

*                                                                      *

*   For GHEISHA:                                                       *

* GPGHEI    returns the hadronic cross section                         *

* GHEISH    Simulates  hadronic  interactions, program  GHEISHA  (H.   *

*           Fesefeldt) in GEANH file.                                  *

*                                                                      *

*   For FLUKA:                                                         *

* FLDIST    returns the hadronic cross section                         *

* FLUFIN    Simulates    hadronic   interactions,    program   FLUKA   *

*           (A.Ferrari et al.) in GEANH file.                          *

*                                                                      *

* MUON INTERACTION                                                     *

*                                                                      *

* GBRELM    Tabulates energy loss by soft Bremsstrahlung               *

* GBRSGM    Tabulates cross-section for hard Bremsstrahlung            *

* GBREMM    Generates hard Bremsstrahlung                              *

* GPAIRI    Tabulates cross-section for direct pair production         *

* GPAIRM    Generate direct pair production                            *

* GMUNUI    Tabulates cross-section for nuclear interaction            *

* GMUNU     Generates nuclear interaction                              *

* GDRELM    Tabulates energy loss by soft delta rays                   *

* GDRSGA    Tabulates cross-section for hard delta rays                *

* GDRAY     Generates delta rays                                       *

*                                                                      *

************************************************************************

#endif