*

* $Id$

*

* $Log: gbase.doc,v $

* Revision 1.1.1.1  2002/06/16 15:18:37  hristov

* Separate distribution  of Geant3

*

* Revision 1.1.1.1  1999/05/18 15:55:16  fca

* AliRoot sources

*

* Revision 1.1.1.1  1995/10/24 10:20:06  cernlib

* Geant

*

*

#include "geant321/pilot.h"

#if defined(CERNLIB_DOC)

*CMZ :  3.21/02 29/03/94  15.41.19  by  S.Giani

*-- Author :

*

************************************************************************

*                                                                      *

*                                                                      *

*                         Introduction to GEANT3                       *

*                         ----------------------                       *

*                                                                      *

*                                                                      *

*    GEANT3 APPLICATIONS                                               *

*                                                                      *

*  The principal applications of GEANT3 are:                           *

*                                                                      *

*  - The  tracking of  particles  through an  experimental setup  for  *

*    acceptance studies or simulation of detector response, and        *

*  - the graphical  representation of the  setup and of  the particle  *

*    trajectories.                                                     *

*                                                                      *

*  It is of course desirable and  very instructive to combine the two  *

*  interactively since the observation of  what happens to a particle  *

*  during the tracking may underline  the weaknesses of the setup and  *

*  makes  the debugging  easier. In view  of these  applications, the  *

*  GEANT3 system allows:                                               *

*                                                                      *

*  - to  describe an  experimental setup  in a  rather efficient  and  *

*    simple  way.   The  setup  is  represented  by  a  structure  of  *

*    geometrical VOLUMEs.  Each volume is  given a 'MEDIUM' number by  *

*    the user.   Different volumes  may have  the same  medium number  *

*    [GEOM].   A  medium is  defined  by  a  set of  parameters,  the  *

*    so-called TRACKING MEDIUM parameters, which include reference to  *

*    the MATERIAL filling the volume [CONS].                           *

*  - to  generate   simulated  events   from  standard   Monte  Carlo  *

*    generators [KINE].                                                *

*  - to  control  the  transport  of particles  through  the  various  *

*    regions of the setup, taking into account the geometrical volume  *

*    boundaries and  all physical  effects due to  the nature  of the  *

*    particles themselves, to their  interactions with the matter and  *

*    to the magnetic field [TRAK, PHYS].                               *

*  - to  record the  elements of  the particle  trajectories and  the  *

*    response from the sensitive detectors [HITS],                     *

*  - to visualize either interactively  or in batch the detectors and  *

*    the particle trajectories [DRAW, XINT].                           *

*                                                                      *

*   Part of the  subroutines available in GEANT3  are integrated into  *

*  program segments which perform these tasks.                         *

*   The  program  segments may  contain  'dummy'  and 'default'  user  *

*  subroutines  called  whenever  application dependent  actions  are  *

*  expected.                                                           *

*   Other  subroutines   provide  tools  either  to   perform  simple  *

*  functions (control  print, debug, I/O,  etc.) or to  implement the  *

*  operations required  for most of the  applications (description of  *

*  the geometrical setup, handling of detector responses,etc.).        *

*   It is the responsibility of  the user to assemble the appropriate  *

*  program segments and tools into an executable program, to code the  *

*  relevant  user subroutines,  to  provide the  data describing  the  *

*  experimental environment and to  submit the appropriate data cards  *

*  which control the  execution of the program.  The  section BASE of  *

*  the User's Guide gives the information necessary to understand how  *

*  to do this job.                                                     *

*                                                                      *

*  Note: as a  general convention the  names of the dummy  or default  *

*        user subroutines have GU or UG  as first two letters and are  *

*        printed in bold characters.                                   *

*                                                                      *

*  EVENT SIMULATION FRAMEWORK                                          *

*                                                                      *

*  The framework for  event simulation is described  in the following  *

*  paragraphs to  familiarize the  reader with  the areas  where user  *

*  interventions are expected.                                         *

*   At  the same  time, the  GEANT3 data  structures are  introduced.  *

*  This last point  is important as the coding to  be provided by the  *

*  user  often consists  of  filling data  structures, or  extracting  *

*  information from  them, or  saving them on  output, making  use of  *

*  standard routines available in the system.                          *

*   A main  program has to  be provided by  the user [BASE  100].  It  *

*  allocates the dynamic memory for ZEBRA and HBOOK and gives control  *

*  to the three phases of the run:                                     *

*                                                                      *

*  - Initialisation                                                    *

*  - Event processing                                                  *

*  - Termination.                                                      *

*                                                                      *

*  INITIALISATION                                                      *

*                                                                      *

*  The initialisation  phase is under  the control of the  user [BASE  *

*  100].  It consists of the  following steps, most of them performed  *

*  via calls to standard GEANT3 subroutines:                           *

*                                                                      *

*  - GINIT,  to  initialize the  GEANT3  common  blocks with  default  *

*    values which the user should be aware of [BASE 030, 110].         *

*  - GFFGO to read  'free format' data cards which  can override some  *

*    of the  values defined in  GINIT the default options  [BASE 040,  *

*    110].                                                             *

*  - GZINIT to initialize the dynamic  core divisions, the link areas  *

*    and the data structure JRUNG [BASE 110].                          *

*  - GDINIT  to initialize the drawing package [DRAW].                 *

*  - GPART  and auxiliaries,  to  generate the  data structure  JPART  *

*    describing the standard particle properties [CONS].               *

*  - GMATE  and auxiliaries,  to  generate the  data structure  JMATE  *

*    describing  the  characteristics  of   the  most  commonly  used  *

*    MATERIALs [CONS].                                                 *

*  - <USER> to define the geometry of the different components of the  *

*    experimental  setup [GEOM]  and the  tracking medium  parameters  *

*    [CONS,TRAK], and  to generate the corresponding  data structures  *

*    JROTM, JVOLUM and JTMED.                                          *

*  - <USER> to specify which elements of the geometrical setup should  *

*    be considered as 'sensitive detectors', giving a 'response' when  *

*    hit by a particle [HITS].                                         *

*  - GGCLOS to close the Geometry package (mandatory).                 *

*  - GBHSTA to  book standard  GEANT3 histograms  as required  by the  *

*    user with the data card HSTA [BASE 040, 110].                     *

*  - GPHYSI to  compute energy loss  and cross section tables  and to  *

*    store them in the data structure JMATE [CONS,PHYS].               *

*                                                                      *

*  EVENT PROCESSING                                                    *

*                                                                      *

*  The processing phase is triggered by  a call to the subroutine GRUN *

*  which,  for each  event to  be  processed,  gives  control to  the  *

*  subroutines:                                                        *

*                                                                      *

*  - GTRIGI, to  initialize the  event processing  and to  create the  *

*    Header bank JHEAD.                                                *

*  - GTRIG to process the event.                                       *

*  - GTRIGC to clean up the event division                             *

*                                                                      *

*  and checks that enough time is left for the next event [BASE 200].  *

*  The main  steps of GTRIG  consist of  calls to the  following user  *

*  routines:                                                           *

*                                                                      *

*  - GUKINE generates the data structures JVERTX and JKINE describing  *

*    the kinematics  of the current  event on input [KINE],  or reads  *

*    them [IOPA].                                                      *

*  - GUTREV (calls GTREVE) controls the  tracking for the whole event  *

*    [TRAK].  Each particle  is tracked in turn and  when a sensitive  *

*    detector is  hit, the user  may store any useful  information in  *

*    the  data  structure JHITS  [HITS].   Before  tracking the  next  *

*    particle, any  secondary products generated by  the current one,  *

*    and stored  by the user  in the temporary data  structure JSTAK,  *

*    are processed in the same way.                                    *

*    Simultaneously,   the  data   structure  JXYZ,   containing  the  *

*    coordinates  of space  points  along the  tracks  for the  whole  *

*    event, can be filled by the user [TRAK].                          *

*  - GUDIGI  simulates the  detector responses  for the  whole event,  *

*    making use  of the information  previously recorded in  the data  *

*    structure JHITS,  and stores the  results in the  data structure  *

*    JDIGI [HITS].                                                     *

*  - GUOUT outputs the relevant data structures for the current event  *

*    [IOPA].                                                           *

*                                                                      *

*  Other user routines called during  the tracking phase triggered by  *

*  GTREVE should be mentioned for completeness:                        *

*                                                                      *

*  - The hadronic processes activated by  default for the tracking of  *

*    hadrons in  GEANT3 are  described by  the program  GHEISHA (file  *

*    GEANH).  In  the subroutines GUPHAD  and GUHADR [TRAK]  the user  *

*    may select, instead of GHEISHA,  the program FLUKA (file GEANF).  *

*  - After each  tracking step of  a given  track in a  given medium,  *

*    control is given to the subroutine GUSTEP.  From the information  *

*    available  in  common  blocks  the  user is  able  to  take  the  *

*    appropriate  action, such  as  storing a  hit or transferring  a  *

*    secondary product  either in  the stack JSTAK  or in  the events  *

*    structure JVERTX/JKINE.                                           *

*  - The subroutine GUSWIM is called  by various tracking routines to  *

*    select the appropriate  code for transport of  the particle over  *

*    the given tracking  step.  A default version is  provided in the  *

*    library for  this routine which in  normal cases need not  to be  *

*    provided by the user.                                             *

*  - The magnetic field,  unless constant with no  X- or Y-component,  *

*    has to be returned by the user subroutine GUFLD.                  *

*                                                                      *

*  TERMINATION                                                         *

*                                                                      *

*   The  termination phase  is under  the control  of the  user [BASE  *

*  300].  For trivial applications it may simply consist of a call to  *

*  the subroutine  GLAST which  computes and prints  some statistical  *

*  information (time per event, use of dynamic memory, etc.).          *

*                                                                      *

*                                                                      *

************************************************************************

*                                                                      *

*                                                                      *

*                  Simplified Program Flow Chart                       *

*                  -----------------------------                       *

*                                                                      *

*  MAIN(user)                                                          *

*  |                                                                   *

*  |-GZEBRA  initialisation   of   ZEBRA  system, dynamic core         *

*  |         allocation                                                *

*  |-UGINIT  (user)                                                    *

*  ||                                                                  *

*  ||- GINIT    initialisation of GEANT3 variables                     *

*  ||- GFFGO    interpretation of data cards                           *

*  ||- GZINIT   initialisation  of ZEBRA  core  divisions and  link    *

*  ||           areas                                                  *

*  ||- GPART    creation of the 'particle' data structure JPART        *

*  ||- GMATE    creation of the 'material' data structure JMATE        *

*  ||- <USER>   description  of the geometrical setup,   of the        *

*  ||           sensitive detectors and creation of data structures    *

*  ||           JVOLUM, JTMED, JROTM, JSETS                            *

*  ||- GGCLOS   close Geometry package                                 *

*  ||- GPHYSI   preparation of cross-sections and energy loss tables   *

*  |            for all used materials                                 *

*  |                                                                   *

*  |-GRUN (loop over events)                                           *

*  ||                                                                  *

*  ||- GTRIGI   initialisation for event processing                    *

*  ||- GTRIG    event processing                                       *

*  ||  |                                                               *

*  ||  |- GUKINE (user)  generation (or input)  of  event initial      *

*  ||  |                 kinematics                                    *

*  ||  |- GUTREV (user)                                                *

*  ||  |   |- GTREVE   (simplified flow for sequential tracking)       *

*  ||  |      |- GSSTAK   store primary tracks in stack                *

*  ||  |      |- Loop over tracks                                      *

*  ||  |        |- GLTRAC   prepare commons for tracking               *

*  ||  |            |- GMEDIA  find current volume /tracking medium    *

*  ||  |        |- GUTRAK (user)                                       *

*  ||  |          |- GTRACK                                            *

*  ||  |            |- GTGAMA/GTELEC/...  tracking   of  particle      *

*  ||  |                                  according to type            *

*  ||  |                 |-   compute physical step size               *

*  ||  |                 |- GTNEXT compute geometrical step size       *

*  ||  |                 |-   propagate (GUSWIM..)                     *

*  ||  |                 |-   test change of volume (GINVOL)           *

*  ||  |            |- GUSTEP (user) recording of hits in data         *

*  ||  |                      structure JHITS and of space points      *

*  ||  |                      in structure JXYZ                        *

*  ||  |- GUDIGI computation  of  digitisations  and recording  in     *

*  ||  |         structure JDIGI                                       *

*  ||  |- GUOUT  output of current event                               *

*  ||                                                                  *

*  ||- GTRIGC   clearing of memory for next event                      *

*  |                                                                   *

*  |-UGLAST (user)                                                     *

*  ||                                                                  *

*  ||- GLAST    standard GEANT3 termination.                           *

*  |                                                                   *

*  |                                                                   *

*  STOP                                                                *

*                                                                      *

*                                                                      *

************************************************************************

************************************************************************

*                                                                      *

*                    Overview of COMMON Blocks                         *

*                    -------------------------                         *

*                                                                      *

*                                                                      *

*  INTRODUCTION                                                        *

*                                                                      *

*  The communication between program segments of the GEANT3 system is  *

*  ensured by data structures and by 'long range' variables in common  *

*  blocks.  In addition,within the  program segments, the subroutines  *

*  communicate with each other  via  explicit arguments  and via  the  *

*  common block variables.                                             *

*   The data structures are described in separate sections. Here, the  *

*  main features of the common  blocks used in GEANT3 are summarized,  *

*  with special mention of the  variables initialized in GINIT and of  *

*  the possibility to override them via data cards [BASE040].          *

*   The  labelled   common  blocks  are  accessible   via  Patchy/CMZ  *

*  sequences identified by the name  of the COMMON.  They are defined  *

*  in the Patch GCDES.                                                 *

*                                                                      *

*  Note: Unless  otherwise specified  the  long  range variables  are  *

*        initialized  in GINIT.   When not  zero, default  values are  *

*        quoted between brackets.  If the value may be modified via a  *

*        standard data  card the  card  keyword is also given between  *

*        brackets.                                                     *

*                                                                      *

*  DYNAMIC CORE                                                        *

*                                                                      *

*  The  GEANT3 data  structures  are stored  in  the common  /GCBANK/  *

*  accessible via the following Patchy sequence.                       *

*                                                                      *

*+KEEP,GCBANK                                                          *

*     PARAMETER (KWBANK=69000,KWWORK=5200)                             *

*     COMMON/GCBANK/NZEBRA,GVERSN,ZVERSN,IXSTOR,IXDIV,IXCONS,FENDQ(16) *

*    +             ,LMAIN,LR1,WS(KWBANK)                               *

*     DIMENSION IQ(2),Q(2),LQ(8000),IWS(2)                             *

*     EQUIVALENCE (Q(1),IQ(1),LQ(9)),(LQ(1),LMAIN),(IWS(1),WS(1))      *

*     EQUIVALENCE (JCG,JGSTAT)                                         *

*     COMMON/GCLINK/JDIGI ,JDRAW ,JHEAD ,JHITS ,JKINE ,JMATE ,JPART    *

*    +      ,JROTM ,JRUNG ,JSET  ,JSTAK ,JGSTAT,JTMED ,JTRACK,JVERTX   *

*    +      ,JVOLUM,JXYZ  ,JGPAR ,JGPAR2,JSKLT                         *

*                                                                      *

*  The /GCLINK/ variables are pointers to the GEANT3 data structures.  *

*  They belong  to a permanent  area declared in GZINIT.   The common  *

*  /GCLINK/ alone may be accessed through the sequence GCLINK.         *

*                                                                      *

*  OTHER LABELLED COMMON BLOCKS                                        *

*                                                                      *

*        COMMON/GCCUTS/CUTGAM,CUTELE,CUTNEU,CUTHAD,CUTMUO,BCUTE,BCUTM) *

*       +             ,DCUTE ,DCUTM ,PPCUTM,TOFMAX,GCUTS(5)            *

*  C                                                                   *

*  CUTGAM   Kinetic energy cut for gammas              (0.001, CUTS)   *

*  CUTELE   Kinetic energy cut for electrons           (0.001, CUTS)   *

*  CUTHAD   Kinetic energy cut for hadrons             (0.01, CUTS)    *

*  CUTNEU   Kinetic energy cut for neutral hadrons     (0.01, CUTS)    *

*  CUTMUO   Kinetic energy cut for muons               (0.01, CUTS)    *

*  BCUTE    Kinetic energy cut for electron Brems.     (CUTGAM, CUTS)  *

*  BCUTM    Kinetic energy cut for muon Brems.         (CUTGAM, CUTS)  *

*  DCUTE    Kinetic energy cut for electron delta-rays (CUTELE, CUTS)  *

*  DCUTM    Kinetic energy cut for muon delta-rays     (CUTELE, CUTS)  *

*  PPCUTM   Kinetic energy cut for e+e- pair production by muons       *

*                                                      (.01, CUTS)     *

*  TOFMAX   Tracking cut on time of flight integrated from  primary    *

*           interaction time                           (1.E+10, CUTS)  *

*  GCUTS    For user applications                      (CUTS)          *

*                                                                      *

*  Note: The cuts BCUTE, BCUTM and DCUTE, DCUTM are given, in GPHYSI,  *

*        the  respective  default  values CUTGAM  and  CUTELE.   Only  *

*        experienced users should make use of the facility offered by  *

*        the data card CUTS to  change BCUTE, DCUTE, BCUTM and DCUTM.  *

*                                                                      *

*     COMMON /GCDRAW/ .....                                            *

*                     see GEANG Pam file                               *

*                                                                      *

*        COMMON/GCFLAG/IDEBUG,IDEMIN,IDEMAX,ITEST,IDRUN,IDEVT,IEORUN   *

*       +        ,IEOTRI,IEVENT,ISWIT(10),IFINIT(20),NEVENT,NRNDM(2)   *

*        COMMON/GCFLAX/BATCH, NOLOG                                    *

*        LOGICAL BATCH, NOLOG                                          *

*  C                                                                   *

*  IDEBUG   Flag set equal to 1 to activate debug if IEVENT (below)    *

*  IDEMIN            is greater or equal to IDEMIN                     *

*  IDEMAX            and less or equal to IDEMAX             (DEBU)    *

*  ITEST    Flag to request printing of IEVENT, IDEVT and NRNDM        *

*           (below) every ITEST events                       (DEBU)    *

*  IDRUN    Current user run number                        (1, RUN)    *

*  IDEVT    Current user event number                         (RUN)    *

*  IEORUN   Flag to terminate current run if non zero                  *

*  IEOTRI   Flag to abort current event if non zero                    *

*  IEVENT   Current event sequence number                       (1)    *

*  ISWIT    Flags reserved for user in relation to debug     (SWIT)    *

*  IFINIT   System flags to check initialisation of GEANT routines     *

*  NEVENT   Number of events to be processed        (10000000,TRIG)    *

*  NRNDM    Initial value of random number seeds NRNDM(1),             *

*           NRNDM(2). If NRNDM(2) is 0, the independent sequence       *

*           NRNDM(1) is used. If NRNDM(1) is 0, the default sequence   *

*           is used. (9876, 54321, RNDM)                               *

*  BATCH    Flag set to .TRUE. if the program is running in            *

*           batch.                                                     *

*  NOLOG    Flag set to .TRUE. if the no logon file has been           *

*           requested.                                                 *

*                                                                      *

*        COMMON/GCJLOC/NJLOC(2),JTM,JMA,JLOSS,JPROB,JMIXT,JPHOT,JANNI  *

*       +                  ,JCOMP,JBREM,JPAIR,JDRAY,JPFIS,JMUNU,JRAYL  *

*       +                  ,JMULOF,JCOEF,JRANG                         *

*  C                                                                   *

*   For relocation of local pointers. Self-explanatory [CONS 199].     *

*                                                                      *

*        COMMON/GCKINE/IKINE,PKINE(10),ITRA,ISTAK,IVERT,IPART,ITRTYP   *

*       +      ,NAPART(5),AMASS,CHARGE,TLIFE,VERT(3),PVERT(4),IPAOLD   *

*  C                                                                   *

*  IKINE    User flag(0, KINE)                                         *

*  PKINE    User array(1E10, KINE)                                     *

*  ITRA     Current track number                                       *

*  ISTAK    Current stack-track number                                 *

*  IVERT    Current vertex number                                      *

*  IPART    Current particle number                                    *

*  ITRTYP   Tracking type of current particle                          *

*  NAPART   Name of current particle                                   *

*  AMASS    Mass of current particle                                   *

*  CHARGE   Charge of current particle                                 *

*  TLIFE    Life-time of current particle                              *

*  VERT     Coordinates of origin vertex for current track             *

*  PVERT    Track kinematics at origin vertex (PVERT(4) no longer      *

*           used)                                                      *

*  IPAOLD   Particle number of the previous track.                     *

*                                                                      *

*        INTEGER MXGKIN                                                *

*        PARAMETER (MXGKIN=100)                                        *

*        COMMON/GCKING/KCASE,NGKINE,GKIN(5,MXGKIN),                    *

*       +                           TOFD(MXGKIN),IFLGK(MXGKIN)         *

*  KCASE     Mechanism having generated the secondary particles        *

*  NGKINE    Number of generated secondaries                           *

*  GKIN(1,I) Px      of I-th secondary                                 *

*  GKIN(2,I) Py               " "                                      *

*  GKIN(3,I) Pz               " "                                      *

*  GKIN(4,I) E                " "                                      *

*  GKIN(5,I) Particle number  " "                                      *

*  TOFD(I)   Time delay introduced by the interaction.                 *

*                                                                      *

*     COMMON/GCLIST/NHSTA,NGET ,NSAVE,NSETS,NPRIN,NGEOM,NVIEW,NPLOT    *

*    +       ,NSTAT,LHSTA(20),LGET (20),LSAVE(20),LSETS(20),LPRIN(20)  *

*    +             ,LGEOM(20),LVIEW(20),LPLOT(20),LSTAT(20)            *

*C                                                                     *

*  NHSTA    Number of histograms declared on data card HSTA            *

*  NGET     Number of data structures declared on data card GET        *

*  NSAVE    Number of data structures declared on data card SAVE       *

*  NSETS    Number of items described on data card SETS                *

*  NPRIN    Number of items described on data card PRIN                *

*  NGEOM    Number of items described on data card GEOM                *

*  NVIEW    Number of items described on data card VIEW                *

*  NPLOT    Number of items described on data card PLOT                *

*  NSTAT    Number of items described on data card STAT                *

*  LHSTA,...,LSTAT Corresponding user lists of items (HSTA,...,STAT)   *

*                                                                      *

*   See examples of utilisation of  the user lists in GEANT3 examples  *

*  in  GEANX file.   LSTAT(1) is  reserved by  the system  for volume  *

*  statistics.                                                         *

*                                                                      *

*        COMMON/GCMATE/NMAT,NAMATE(5),A,Z,DENS,RADL,ABSL               *

*  C                                                                   *

*  NMAT      Current material number                                   *

*  NAMATE    Name of current material                                  *

*  A         Atomic weight of current material                         *

*  Z         Atomic number of current material                         *

*  DENS      Density of current material                               *

*  RADL      Radiation length of current material                      *

*  ABSL      Absorption length of current material                     *

*                                                                      *

*     COMMON/GCMULO/SINMUL(101),COSMUL(101),SQRMUL(101),OMCMOL,CHCMOL  *

*    +  ,EKMIN,EKMAX,NEKBIN,NEK1,EKINV,GEKA,GEKB,EKBIN(200),ELOW(200)  *

*                                                                      *

*   Pre-computed  quantities  for   multiple  scattering  and  energy  *

*  binning [CONS 199]                                                  *

*  SINMUL   Not used                                                   *

*  COSMUL   Not used                                                   *

*  SQRMUL   Not used                                                   *

*  OMCMOL   Constant Omega for Moliere scattering                      *

*  CHCMOL   Constant for Moliere scattering                            *

*  EKMIN    Lower edge for the energy range           (1E-5, ERAN)     *

*  EKMAX    Upper edge for the energy range           (1E+4, ERAN)     *

*  NEKBIN   Number of energy bins to be used          (90, ERAN)       *

*  NEK1     Number of energy bins to be used + 1                       *

*  EKINV    \                                                          *

*  GEKA      >Constants for the energy binning                         *

*  GEKB     /                                                          *

*  EKBIN    Lower edges of energy bins                                 *

*  ELOW     Lower edges of logarithm of energy bins                    *

*                                                                      *

*        COMMON/GCNUM/NMATE ,NVOLUM,NROTM,NTMED,NTMULT,NTRACK,NPART    *

*       +            ,NSTMAX,NVERTX,NHEAD,NBIT                         *

*        COMMON /GCNUMX/ NALIVE,NTMSTO                                 *

*  C                                                                   *

*  NMATE    Number of Materials                                        *

*  NVOLUM   Number of Volumes                                          *

*  NROTM    Number of Rotation matrices                                *

*  NTMED    Number of Tracking media                                   *

*  NTMULT   Number of  tracks processed  in current  event (including  *

*           secondaries), reset to 0 for each event                    *

*  NTRACK   Number of tracks in JKINE banks for current event          *

*  NPART    Number of Particle banks                                   *

*  NSTMAX   Maximum  number of  tracks  in  stack JSTAK  for  current  *

*           event, reset to 0 for each event                           *

*  NVERTX   Number of Vertices in JVERTX banks for current event       *

*  NHEAD    Number of data words in the JHEAD bank       (10)          *

*  NBIT     Number of bits per word (initialized in GINIT via FFINIT   *

*  NALIVE   Internal counter used for parallel tracking                *

*  NTMSTO   Internal counter used for parallel tracking                *

*                                                                      *

**KEEP,GCONSP                                                          *

*      DOUBLE PRECISION PI,TWOPI,PIBY2,DEGRAD,RADDEG,CLIGHT,BIG,EMASS  *

*      DOUBLE PRECISION EMMU,PMASS,AVO                                 *

*C                                                                     *

*      PARAMETER (PI=3.14159265358979324)                              *

*      PARAMETER (TWOPI=6.28318530717958648)                           *

*      PARAMETER (PIBY2=1.57079632679489662)                           *

*      PARAMETER (DEGRAD=0.0174532925199432958)                        *

*      PARAMETER (RADDEG=57.2957795130823209)                          *

*      PARAMETER (CLIGHT=29979245800.)                                 *

*      PARAMETER (BIG=10000000000.)                                    *

*      PARAMETER (EMASS=0.0005109990615)                               *

*      PARAMETER (EMMU=0.105658387)                                    *

*      PARAMETER (PMASS=0.9382723128)                                  *

*      PARAMETER (AVO=0.60221367)                                      *

*                                                                      *

*  PI       Number PI                                                  *

*  TWOPI    (2.*PI)                                                    *

*  PIBY2    (PI/2.)                                                    *

*  DEGRAD   Degree to radian conversion factor  (PI/180.)              *

*  RADDEG   Radian to degree conversion factor  (180./PI)              *

*  CLIGHT   Light velocity                                             *

*  BIG      Arbitrary large number                                     *

*  EMASS    Electron mass                                              *

*  EMMU     Muon mass                                                  *

*  PMASS    Proton mass                                                *

*  AVO      Avogadro Number * 1E-24                                    *

*                                                                      *

*  Control of Geometry optimisation                                    *

*     COMMON/GCOPTI/IOPTIM                                             *

*  IOPTIM  -1 = No optimisation at all. GSORD calls disabled           *

*           0 = No optimisation. Only user calls to GSORD kept         *

*           1 = All non-ordered volumes are ordered along the best axis*

*           2 = All volumes are ordered along the best axis            *

*                                                                      *

*   Control of physics processes.                                      *

*        COMMON/GCPHYS/IPAIR,SPAIR,SLPAIR,ZINTPA,STEPPA                *

*       +             ,ICOMP,SCOMP,SLCOMP,ZINTCO,STEPCO                *

*       +             ,IPHOT,SPHOT,SLPHOT,ZINTPH,STEPPH                *

*       +             ,IPFIS,SPFIS,SLPFIS,ZINTPF,STEPPF                *

*       +             ,IDRAY,SDRAY,SLDRAY,ZINTDR,STEPDR                *

*       +             ,IANNI,SANNI,SLANNI,ZINTAN,STEPAN                *

*       +             ,IBREM,SBREM,SLBREM,ZINTBR,STEPBR                *

*       +             ,IHADR,SHADR,SLHADR,ZINTHA,STEPHA                *

*       +             ,IMUNU,SMUNU,SLMUNU,ZINTMU,STEPMU                *

*       +             ,IDCAY,SDCAY,SLIFE ,SUMLIF,DPHYS1                *

*       +             ,ILOSS,SLOSS,SOLOSS,STLOSS,DPHYS2                *

*       +             ,IMULS,SMULS,SOMULS,STMULS,DPHYS3                *

*       +             ,IRAYL,SRAYL,SLRAYL,ZINTRA,STEPRA                *

*  *                                                                   *

*  IPAIR    Controls pair production process               (1,PAIR)    *

*           0 = no pair production                                     *

*           1 = pair production with generation of secondaries         *

*           2 = same without generation of secondaries                 *

*  ICOMP    Controls Compton scattering process            (1,COMP)    *

*           0 = no Compton scattering                                  *

*           1 = Compton scattering with generation of secondaries      *

*           2 = same without generation of secondaries                 *

*  IPHOT    Controls photo-electric effect process         (1,PHOT)    *

*           0 = no photo-electric effect                               *

*           1 = photo-electric effect with generation of secondaries   *

*           2 = same without generation of secondaries                 *

*  IPFIS    Controls photofission process                  (0,PFIS)    *

*           0 = no photofission                                        *

*           1 = photofission with generation of secondaries            *

*           2 = same without generation of secondaries                 *

*  IDRAY    Controls delta rays process                    (1,DRAY)    *

*           0 = no delta rays effect                                   *

*           1 = delta rays with generation of secondaries              *

*           2 = same without generation of secondaries                 *

*  IANNI    Controls positron annihilation process         (1,ANNI)    *

*           0 = no positron annihilation effect                        *

*           1 = positron annihilation with generation of secondaries   *

*           2 = same without generation of secondaries                 *

*  IBREM    Controls Bremsstrahlung process                (1,BREM)    *

*           0 = no Bremsstrahlung effect                               *

*           1 = Bremsstrahlung with generation of secondaries          *

*           2 = same without generation of secondaries                 *

*  IHADR    Controls hadron interactions process           (1,HADR)    *

*           0 = no hadron interactions effect                          *

*           1 = hadron interactions with generation of secondaries     *

*           2 = same without generation of secondaries                 *

*  IMUNU    Controls muon nuclear interaction process       (0,MUNU)   *

*           0 = no muon nuclear interaction effect                     *

*           1 = muon nuclear interaction with generation of secondaries*

*           2 = same without generation of secondaries                 *

*  IDCAY    Controls decay process                          (1,DCAY)   *

*           0 = no decay effect                                        *

*           1 = decay with generation of secondaries                   *

*           2 = same without generation of secondaries                 *

*  ILOSS    Controls energy loss process                    (2,LOSS)   *

*           0 = no energy loss effect                                  *

*           1 = delta ray and reduced Landau fluctuations              *

*           2 = full Landau fluctuations and no delta rays             *

*           3 = same as 1                                              *

*           4 = average Energy loss and no fluctuations                *

*  IMULS    Controls multiple scattering                   (1,MULS)    *

*           1 = Moliere or Coulomb scattering                          *

*           2 = Moliere or Coulomb scattering                          *

*           3 = Gaussian scattering                                    *

*  IRAYL    0 = No Rayleigh scattering                                 *

*           1 = Rayleigh scattering                                    *

*                                                                      *

*        COMMON/GCPOLY/IZSEC,IPSEC                                     *

*  C                                                                   *

*  Internal flags for polygon and polycone shapes. See GEANG file.     *

*                                                                      *

*        COMMON/GCPUSH/NCVERT,NCKINE,NCJXYZ,NPVERT,NPKINE,NPJXYZ       *

*  C                                                                   *

*  NCVERT   Initial size of mother bank JVERTX              (5)        *

*  NCKINE   Initial size of mother bank JKINE              (50)        *

*  NCJXYZ   Initial size of mother bank JXYZ               (50)        *

*  NPVERT   Increment for size of mother bank JVERTX        (5)        *

*  NPKINE   Increment for size of mother bank JKINE        (10)        *

*  NPJXYZ   Increment for size of mother bank JXYZ         (10)        *

*                                                                      *

*        COMMON/GCSETS/IHSET,IHDET,ISET,IDET,IDTYPE,NVNAME,NUMBV(20)   *

*  C                                                                   *

*  IHSET    Set identifier                                             *

*  IHDET    Detector identifier                                        *

*  ISET     Position of set in bank JSET                               *

*  IDET     Position of detector in bank JS=IB(JSET-IDET)              *

*  IDTYPE   User defined detector type                                 *

*  NVNAME   Number of elements in NUMBV                                *

*  NUMBV    List of volume numbers to identify the detector            *

*                                                                      *

*      PARAMETER (NWSTAK=12,NWINT=11,NWREAL=12,NWTRAC=NWINT+NWREAL+5)  *

*      COMMON /GCSTAK/ NJTMAX, NJTMIN, NTSTKP, NTSTKS, NDBOOK, NDPUSH, *

*     +                NJFREE, NJGARB, NJINVO, LINSAV(15), LMXSAV(15)  *

*C                                                                     *

*  NTKSTP   Primary allocation for stack JSTAK                         *

*  NTKSTS   Secondary allocation for stack JSTAK                       *

*  NDBOOK   local variable for control of stack size                   *

*  NDPUSH   local variable for control of stack size                   *

*  (other variables used in parallel tracking only)                    *

*                                                                      *

*        COMMON/GCTIME/TIMINT,TIMEND,ITIME,IGDATE,IGTIME               *

*        INTEGER ITIME,IGDATE,IGTIME                                   *

*        REAL TIMINT,TIMEND                                            *

*  C                                                                   *

*  TIMINT   Total time left after initialization  (System, TIME)       *

*  TIMEND   Time required for program termination phase (1., TIME)     *

*  ITIME    Test on time left done every ITIME events (1, ITIME)       *

*  IGDATE   Date of the day  YYMMDD  integer (e.g. 920407)             *

*  IGTIME   Time of the day  HHMM    integer (e.g. 1425)               *

*                                                                      *

*      COMMON/GCTMED/NUMED,NATMED(5),ISVOL,IFIELD,FIELDM,TMAXFD,STEMAX *

*     +      ,DEEMAX,EPSIL,STMIN,CFIELD,PREC,IUPD,ISTPAR,NUMOLD        *

*  NUMED    Current tracking medium number                             *

*  NATMED   Name of current tracking medium                            *

*  ISVOL    Sensitive volume flag (if non zero)                        *

*  IFIELD   Field map type (0 if no field)                             *

*  FIELDM   Maximum field                                              *

*  TMAXFD   Maximum field turning angle in one step                    *

*  STEMAX   Maximum step allowed                                       *

*  DEEMAX   Maximum energy loss gradient in one step                   *

*  EPSIL    Boundary crossing accuracy                                 *

*  STMIN    Minimum step size by energy loss, multiple scattering      *

*           or field                                                   *

*  CFIELD   Constant for field step evaluation                         *

*  PREC     Initial step for boundary crossing (0.1*EPSIL)             *

*  IUDP     0 If medium change, (1 otherwise)                          *

*  ISTPAR   0 If standard tracking parameters                          *

*  NUMOLD   Numed of the last medium.                                  *

*                                                                      *

*        PARAMETER (MAXMEC=30)                                         *

*        COMMON/GCTRAK/VECT(7),GETOT,GEKIN,VOUT(7),NMEC,LMEC(MAXMEC)   *

*       + ,NAMEC(MAXMEC),NSTEP ,MAXNST,DESTEP,DESTEL,SAFETY,SLENG      *

*       + ,STEP  ,SNEXT ,SFIELD,TOFG  ,GEKRAT,UPWGHT,IGNEXT,INWVOL     *

*       + ,ISTOP ,IGAUTO,IEKBIN, ILOSL, IMULL,INGOTO,NLDOWN,NLEVIN     *

*       + ,NLVSAV,ISTORY                                               *

*  VECT     Current track parameters (X,Y,Z,Px/P,Py/P,Pz/P,P)          *

*  GETOT    Current track total energy                                 *

*  GEKIN    Current track kinetic energy                               *

*  VOUT     Local use                                                  *

*  NMEC     Number of mechanisms for current step                      *

*  LMEC     List of mechanism indices for current step                 *

*  NAMEC    Mechanism names (See below)                                *

*  NSTEP    Number of steps so far                                     *

*  MAXNST   Maximum number of steps allowed (default = 10000)          *

*  DESTEP   Total energy lost in current step                          *

*  DESTEL   Continuous energy loss in current step                     *

*  SAFETY   Overestimated distance to closest medium boundary          *

*  SLENG    Track length at current point                              *

*  STEP     Size of current tracking step                              *

*  SNEXT    Straight distance to next current medium boundary          *

*  SFIELD   Field turning angle step size evaluation                   *

*  TOFG     Time of flight                                             *

*  GEKRAT   Interpolation factor in table ELOW                         *

*  UPWGHT   User particle weight                                       *

*  IGNEXT   Flag set to 1 when SNEXT has been recomputed               *

*  INWVOL   Flag set to 1 when entering a new volume,                  *

*                       2 when leaving a volume and                    *

*                       3 when leaving the experimental setup.         *

*                       0 otherwise 0                                  *

*  ISTOP    Flag set to 1 when track looses its identity               *

*                       2 when energy below cut                        *

*  IGAUTO   Automatic computation of DEEMAX,STMIN,TMAXFD,STEMAX        *

*  IEKBIN   Current kinetic energy bin in table ELOW                   *

*  ILOSL    Local value of ILOSS for current tracking medium           *

*  IMULL    Local value of IMULS for current tracking medium           *

*  INGOTO   Content number of limiting content when computing SNEXT    *

*  NLDOWN   Lowest level reached down the tree (parallel tracking only)*

*  NLEVIN   Number of levels currently filled and valid in /GCVOLU/    *

*  NLVSAV   Current level (parallel tracking only)                     *

*  ISTORY   User flag for current track history (reset to 0 in GLTRAC) *

*  --------                                                            *

*  NAMEC    List  of possible  mechanisms  for  step size  limitation  *

*           filled in GINIT :                                          *

*     DATA MEC/'NEXT','MULS','LOSS','FIEL','DCAY','PAIR','COMP','PHOT' *

*    +        ,'BREM','DRAY','ANNI','HADR','ECOH','EVAP','FISS','ABSO' *

*    +        ,'ANNH','CAPT','EINC','INHE','MUNU','TOFM','PFIS','SCUT' *

*    +        ,'RAYL','PARA','PRED','LOOP','NULL','STOP'/              *

*  *                                                                   *

*                                                                      *

*        COMMON/GCUNIT/LIN,LOUT,NUNITS,LUNITS(5)                       *

*        INTEGER LIN,LOUT,NUNITS,LUNITS                                *

*        COMMON/GCMAIL/CHMAIL                                          *

*        CHARACTER*132 CHMAIL                                          *

*  C                                                                   *

*  LIN      Input unit to read data cards                              *

*  LOUT     Line printer output unit                                   *

*  NUNITS   Number of additional units                                 *

*  LUNITS   List of additional units.                                  *

*  CHMAIL   Internal string used for error messages                    *

*   LIN and  LOUT are defined in  GINIT through calls to  the routine  *

*  FFGET from  the standard  FFREAD package.   NUNITS and  LUNITS are  *

*  reserved for user applications.                                     *

*                                                                      *

*     COMMON /GCVOLU/   ......                                         *

*                     see GEANG Pam file                               *

*                                                                      *

*                                                                      *

*                                                                      *

************************************************************************

*                                                                      *

*                      Summary of Data Cards                           *

*                      ---------------------                           *

*                                                                      *

*                                                                      *

*  INTRODUCTION                                                        *

*                                                                      *

*  GEANT3 uses the standard FFREAD package to read 'free format' data  *

*  cards in  the routine GFFGO.   The cards currently  interpreted by  *

*  GFFGO can be classified into four categories:                       *

*                                                                      *

*  - General control of the run.                                       *

*  - Control of the physics processes.                                 *

*  - Debug and I/O operations.                                         *

*  - User applications.                                                *

*                                                                      *

*  The data  cards are  listed below by  category with  the following  *

*  information:                                                        *

*                                                                      *

*  - KEY, card  keyword, any  number of  characters truncated  to the  *

*    first 4                                                           *

*  - N, maximum expected number of variables NVAR,                     *

*  - T,  TYPE of these variables (I=INTEGER,  R=REAL or M=MIXED)       *

*                                                                      *

*  for each variable in turn:                                          *

*                                                                      *

*      - VAR.., FORTRAN name                                           *

*      - Short description (more detail in BASE 030)                   *

*      - COMMON where it is stored, and                                *

*      - Default value from GINIT.                                     *

*                                                                      *

*  When a  card is decoded,  the values  entered by the  user without  *

*  explicit assignment are  assigned to the variables  in order.  The  *

*  number of values can  be less than NVAR.  In case  of a MIXED type  *

*  the values entered have to be in agreement with the default of the  *

*  corresponding FORTRAN variable names.                               *

*                                                                      *

*  Example of data card:     RUN   5   201                             *

*                                                                      *

*  to  preset  the  current  run  and  event  number  to  5  and  201  *

*  respectively.                                                       *

*   In batch jobs  there is no need for any  special termination card  *

*  and none of the cards mentioned below is mandatory.                 *

*                                                                      *

*  USER DEFINED DATA CARDS                                             *

*                                                                      *

*  Before  calling  GFFGO the  user  may  define private  data  cards  *

*  through calls to FFKEY as follows:                                  *

*      CALL FFKEY('KEY',VAR(1),NVAR,'TYPE')                            *

*   They will be interpreted by GFFGO in the same way as the standard  *

*  cards.                                                              *

*                                                                      *

*  SUMMARY OF THE MOST IMPORTANT GEANT3 DATA CARDS                     *

*                                                                      *

*  KEY   N  T  VAR..  Short description                COMMON  GINIT   *

*  General control of the run:                                         *

*  HSTA 20  M LHSTA   Names of required histograms     GCLIST    0     *

*  PATR  4  I NJTMAX  Max number of tracks in parallel GCSTAK    0     *

*                     tracking stack                                   *

*             NJTMIN  Number of tracks above which        "      0     *

*                     parallel tracking can be                         *

*                     reactivated when frozen earlier                  *

*             NTSTKP  Primary allocation for stack JSTAK  "    500     *

*             NTSTKS  Secondary  ... (when parallel       "    100     *

*                     tracking used)                                   *

*  RNDM  2  I NRNDM   Initial random number seeds                      *

*             NRNDM(1)                                 GCFLAG  9876    *

*             NRNDM(2)                                 GCFLAG 54321    *

*  RNDM  2  I NRNDM   Initial random number seeds      GCFLAG    0     *

*  RUNG  2  I IDRUN   User run number                  GCFLAG    1     *

*             IDEVT   User event number                GCFLAG    1     *

*  TIME  3  M TIMINT  Time left after initialisation   GCTIME System   *

*             TIMEND  Time required for termination    GCTIME   1.     *

*             ITIME   Test every ITIME events          GCTIME   1      *

*  TRIG  1  I NEVENT  Number of events to process      GCFLAG  1E7     *

*  Geometry optimization:                                              *

*  OPTI  1  I IOPTIM  Optimization level               GCOPTI   1      *

*  SCAN process control:                                               *

*  SCAN  8  M         SCAN granularity and mode                        *

*           SCANFL    Scan processing flag (Logical)   GCSCAN  FALSE   *

*           NPHI      Number of divisions in PHI       GCSCAN   90     *

*           PHIMIN    Minimum value of PHI             GCSCAN   0.     *

*           PHIMAX    Maximum value of PHI             GCSCAN 360.     *

*           NTETA     Number of divisions in TETA      GCSCAN  90      *

*           TETMIN    Minimum value of TETA            GCSCAN  0.      *

*           TETMAX    Maximum value of TETA            GCSCAN 180.     *

*           MODTET    Type of TETA division            GCSCAN   1      *

*  SCAL 32  M SLIST   List of scanned volumes          GCSCAN  'XXXX'  *

*  SCAP  6  R         SCAN parameters                                  *

*           VX      SCAN vertex X coordinate           GCSCAN  0.0     *

*           VY      SCAN vertex Y coordinate           GCSCAN  0.0     *

*           VZ      SCAN vertex Z coordinate           GCSCAN  0.0     *

*           FACTX0  Scale factor for SX0               GCSCAN  100.    *

*           FACTL   Scale factor for SL                GCSCAN  1000.   *

*           FACTR   Scale factor for R                 GCSCAN  100.    *

*  Control of physics processes:                                       *

*  AUTO  1  I IGAUTO  Automatic computation of STMIN   GCTRAK   1      *

*                     STEMAX,DEEMAX,TMAXFD                             *

*                     0 = Tracking media parameters                    *

*                         taken from the argument list                 *

*                         of GSTMED                                    *

*                     1 = Tracking media parameters                    *

*                         calculated by GEANT                          *

*  ANNI  1  I IANNI   Annihilation flag                GCPHYS   1      *

*  BREM  1  I IBREM   Bremsstrahlung flag              GCPHYS   1      *

*  COMP  1  I ICOMP   Compton scattering flag          GCPHYS   1      *

*  CUTS 15  R         Kinetic energy cuts :                            *

*           CUTGAM    " "   for gammas                 GCCUTS   0.001  *

*           CUTELE    " "   for electrons              GCCUTS   0.001  *

*           CUTHAD    " "   for charged hadrons        GCCUTS   0.01   *

*           CUTNEU    " "   for neutral hadrons        GCCUTS   0.01   *

*           CUTMUO    " "   for muons                  GCCUTS   0.01   *

*           BCUTE     " "   for electron brems.        GCCUTS   CUTGAM *

*           BCUTM     " "   for muon Brems.            GCCUTS   CUTGAM *

*           DCUTE     " "   for electron delta-rays    GCCUTS   CUTELE *

*           DCUTM     " "   for muon delta-rays        CCUTS    CUTELE *

*           PPCUTM    " "   for e+e- pairs by muons    CCUTS    10 MeV *

*           TOFMAX  Time of flight cut                 GCCUTS   1.E+10 *

*           GCUTS   5 user words                       GCCUTS   0.     *

*  DCAY  1  I IDCAY   Decay flag                       GCPHYS   1      *

*  DRAY  1  I IDRAY   delta-rays flag                  GCPHYS   1      *

*  ERAN  3  M Cross section tables                                     *

*           R EKMIN   Minimum energy for the tables    GCMULO  1E-5    *

*           R EKMAX   Maximum energy for the tables    GCMULO  1E+4    *

*           I NEKBIN  Number of bins in the table      GCMULO   90     *

*  HADR  1  I IHADR   Hadronic process flag            GCPHYS   1      *

*  LOSS  1  I ILOSS   Energy loss flag                 CGPHYS   2      *

*  MULS  1  I IMULS   Multiple scattering flag         GCPHYS   1      *

*  MUNU  1  I IMUNU   Muon nuclear interactions flag   GCPHYS   0      *

*  PAIR  1  I IPAIR   Pair production flag             GCPHYS   1      *

*  PFIS  1  I IPFIS   Photofission flag                GCPHYS   0      *

*  PHOT  1  I IPHOT   Photo-electric effect flag       GCPHYS   1      *

*  RAYL  1  I IRAYL   Rayleigh scattering flag         GCPHYS   0      *

*                                                                      *

*  Debug and I/O operations:                                           *

*  DEBU 3  M IDEMIN  First event to debug              GCFLAG   0      *

*            IDEMAX  Last event to debug               GCFLAG   0      *

*            ITEST   Print control frequency           GCFLAG   0      *

*  GET  20 M LGET    Names of data structure to get    GCLIST   ' '    *

*  PRIN 20 M LPRIN   User keywords to print data       GCLIST   ' '    *

*                   structures                                         *

*  SAVE 20 M LSAVE   Names of data struct. to save     GCLIST   ' '    *

*  SWIT 10 I ISWIT   User flags for debug or else      GCFLAG   0      *

*                   User applications:                                 *

*  KINE 11 M IKINE   User flag                         GCKINE   0      *

*            PKINE   10 user words                     GCKINE 1.E+10   *

*  SETS 20 M LSETS   User words for detector sets      GCLIST   ' '    *

*  STAT 20 M LSTAT   1 system + 19 user words          GCLIST   ' '    *

*  PLOT 20 M LPLOT   User words to control plots       GCLIST   ' '    *

*  GEOM 20 M LGEOM   User words to control geometry    GCLIST   ' '    *

*  VIEW 20 M LVIEW   User words to control View banks  GCLIST ' '      *

*                                                                      *

*                                                                      *

*                                                                      *

*                                                                      *

*                                                                      *

*                                                                      *

************************************************************************

*                                                                      *

*               The Reference Systems and dimensional Units            *

*               -------------------------------------------            *

*                                                                      *

*                                                                      *

*  THE MASTER REFERENCE SYSTEM  (MARS)                                 *

*                                                                      *

*   The Master Reference  System (MARS) is determined by  the way the  *

*  user  represents  the kinematical  quantities.   If  the axes  are  *

*  labelled (X,Y,Z), then the point P(A,B,C) is represented by         *

*                                                                      *

*           Y |                                                        *

*             |         * P(A,B,C)                                     *

*             |                    * X                  A on axis X    *

*             |                  *                      B on axis Y    *

*             |                *                        C on axis Z    *

*             |              *                                         *

*             |            *                                           *

*             |          *                                             *

*             |        *                                               *

*             |      *                                                 *

*             |                                                        *

*             |                                                        *

*             ............................>                            *

*                                         Z                            *

*    The tracking is performed in  the MAster Reference System.  This  *

*  implies that  the arguments  of the  user magnetic  field routine,  *

*  space point  coordinates and field  components, are given  in this  *

*  system.                                                             *

*                                                                      *

*  THE LOCAL REFERENCE SYSTEMS (MRS AND DRS)                           *

*                                                                      *

*   As explained in GEOM 001, the experimental set-up is described by  *

*  the  definition  of  an   'initial  MOTHER'  volume  inside  which  *

*  'DAUGHTER' volumes are positioned.   Other daughter volumes can be  *

*  positioned  inside  these volumes  which  are  promoted as  mother  *

*  volumes and so on, as russian dolls.                                *

*   This  requires the  definition  of local  reference systems,  the  *

*  Mother  Reference  Systems  (MRS,  Origin  O.)  and  the  Daughter  *

*  Reference Systems (DRS, Origin O.).                                 *

*   The  local  reference  system  of  the  'initial  mother'  volume  *

*  coincides with the MAster Reference System.                         *

*   The full description of a given  detector is usually given in the  *

*  local reference system of the associated volume.                    *

*   The transformation of a point from  the MRS (V.) to the DRS (V.),  *

*  at any level, requires the knowledge  of a rotation matrix R and a  *

*  translation vector T defined through the relation :                 *

*              ( V. ) = [ R ] ( V. - T )                               *

*   The components of  T are the projections of the  vector O.O. onto  *

*  the MRS axes.                                                       *

*   The rotation matrices  are computed from the  spherical angles of  *

*  each of  the axes of  the daughter  reference system (I,  II, III)  *

*  with respect to the mother reference system (1,2,3).                *

*   The spherical angles  THETA and PHI of a direction  D are defined  *

*  as follows :                                                        *

*                                                                      *

*  THETA     is the angle formed by the axis 3 and D (range : 0 to 180 *

*            degrees)                                                  *

*  PHI       is the angle formed by the axis 1 and the projection of D *

*            onto the plane defined by the axes 1 and 2 (range : 0 to  *

*            360 degrees)                                              *

*  Examples are given in GEOM 200.                                     *

*   The various rotation matrices required for a given set-up must be  *

*  defined  during  the initialisation  stage,  usually  in the  user  *

*  routine UGEOM.                                                      *

*   A serial number is assigned to each matrix [GEOM 200].             *

*   The translation parameters and the  serial number of the rotation  *

*  matrix are specified  by the user when the  volumes are positioned  *

*  inside the set-up [GEOM 110].                                       *

*                                                                      *

*  THE DIMENSIONAL UNITS                                               *

*                                                                      *

*  Unless  otherwise   specified,  the   following  units   are  used  *

*  throughout the program :                                            *

*                                                                      *

*      -  CENTIMETER, SECOND, KILOGAUSS, GEV, GEV/C, DEGREE            *

*                                                                      *

************************************************************************

*                                                                      *

*        Examples of MAIN Program and User Initialisation              *

*        ------------------------------------------------              *

*                                                                      *

*      PROGRAM MAIN                                                    *

*  C                                                                   *

*      PARAMETER (NG=100000,NH=10000)                                  *

*      COMMON/PAWC/H(NH)                                               *

*      COMMON/GCBANK/Q(NG)                                             *

*  C                                                                   *

*  C       Allocate memory for ZEBRA and HBOOK                         *

*      CALL GZEBRA(NG)                                                 *

*      CALL HLIMIT(-NH)                                                *

*  C                                                                   *

*  C       Initialize Graphics package                                 *

*      CALL IGINIT(0)                                                  *

*  C       Open metafile and define workstation type                   *

*  C         (computer dependent)                                      *

*                  ....                                                *

*  C                                                                   *

*  C       Initialisation phase                                        *

*      CALL UGINIT                                                     *

*  C                                                                   *

*  C       Processing phase                                            *

*      CALL GRUN                                                       *

*  C                                                                   *

*  C       Termination phase                                           *

*      CALL UGLAST                                                     *

*  C                                                                   *

*      END                                                             *

*      SUBROUTINE UGINIT                                               *

*  C                                                                   *

*  +SEQ,GCLIST                                                         *

*  C                                                                   *

*  C       Initialize GEANT variables                                  *

*      CALL GINIT                                                      *

*  C                                                                   *

*  C       Read data cards                                             *

*      CALL GFFGO                                                      *

*  C                                                                   *

*  C       Initialize data structures                                  *

*      CALL GZINIT                                                     *

*  C                                                                   *

*  C       Initialize drawing package                                  *

*      CALL GDINIT                                                     *

*  C                                                                   *

*  C       Open I/O buffers                                            *

*      IF(NGET .GT.0)CALL GOPEN(1,'I',0,IER)                           *

*      IF(NSAVE.GT.0)CALL GOPEN(2,'O',0,IER)                           *

*  C                                                                   *

*  C       Fetch permanent data structures (if any)                    *

*      CALL GFIN(1,'INIT',1,IDENT,' ',IER)                             *

*      IF(IER.LT.0) THEN                                               *

*  C                                                                   *

*  C       Define standard Particle and Material data                  *

*         CALL GPART                                                   *

*         CALL GMATE                                                   *

*  C                                                                   *

*  C       Define the geometrical set-up                               *

*         CALL 'user code'                                             *

*         CALL GGCLOS                                                  *

*  C                                                                   *

*  C       Compute cross-section and energy loss tables                *

*         CALL GPHYSI                                                  *

*       ENDIF                                                          *

*  C                                                                   *

*  C       Initialize standard histograms                              *

*       CALL GBHSTA                                                    *

*  C                                                                   *

*       END                                                            *

*                                                                      *

*                                                                      *

*                                                                      *

************************************************************************

*                                                                      *

*               The System Initialisation routines                     *

*               ----------------------------------                     *

*                                                                      *

*  Presets COMMON  block variables to default  values.  Preprocessing  *

*  of  various  COMMON  block  variables.  See  'Overview  of  COMMON  *

*  blocks' [BASE  030].  Reads a  set of  data cards with  the FFREAD  *

*  package.  See 'Summary  of data cards' [BASE 040]  GFFGO should be  *

*  called  after  GINIT.   Allocates   the  dynamic  core  divisions.  *

*  Initialize the link areas and the data structure JRUNG [BASE 299].  *

*  Initialize  exotic bank  formats.  GZINIT  should be  called after  *

*  GFFGO.  To be called before the  user geometry routine if the user  *

*  wants  to  open  VIEW   banks  there.   Initializes  any  standard  *

*  histogram required by the user with the data record HSTA.           *

*   The following histogram keywords may be used :                     *

*  TIME     Time per event                                             *

*  SIZE     Size of division LXDIV per event                           *

*  MULT     Total number of tracks per event                           *

*  NTRA     Number of 'long life' tracks per event                     *

*  STAK     Maximum stack size per event                               *

*                                                                      *

*   GBHSTA should be called after GFFGO.                               *

*                                                                      *

*             Steering routines for Event Processing                   *

*             --------------------------------------                   *

*                                                                      *

*   The following flow chart is  only valid for the 'batch' execution  *

*  mode.   For  interactive  applications, see  section  XINT.   Main  *

*  routine to control a run of events                                  *

*                     .........                                        *

*                 ...>|  TIMEX|                                        *

*                 |   .........                                        *

*                 |      Get    time   left    (TIMINT)    after       *

*   initialisation                                                     *

*                 |                                                    *

*                 .............................                        *

*                 |    ...........            |                        *

*                 ... >| GTRIGI  |            |                        *

*                 |    ...........            |                        *

*                 |    ...........            |                        *

*   ........ .    |.. >| GTRIG   |       ......... .                   *

*   |        |    |    ......... .       |loop on  |                   *

*   | GRUN   |... |    |                 |         |                   *

*   ........ .    |    ......... .       | events  |                   *

*                 |.. >| GTRIGC  |       ......... .                   *

*                 |    ......... .            .                        *

*                 |    ...................    |                        *

*                 ... >| check time left |    |                        *

*                 |    ...................    |                        *

*                 .............................                        *

*                                                                      *

*   Resets to 0  the flag IEOTRI in /GCFLAG/ and  the counters NTRACK  *

*  and NVERTX in /GCNUM/.  Sets the  debug flag IDEBUG in /GCFLAG/ to  *

*  the value required for the current event.                           *

*   Creates the Header bank for current event.                         *

*   Prints  the sequence  number,  the event  number  and the  number  *

*  random  generators, under  control of  the flag  ITEST (data  card  *

*  DEBU).                                                              *

*   Steering routine to process one event (trigger)                    *

*                                                                      *

*                      ..........................                      *

*                      | Generates kinematics,  |                      *

*                 ... >| or read event GUKINE   |                      *

*                 |    ..........................                      *

*                 |    ................                                *

*                 |    | Tracking/hits|                                *

*   ........ .    |.. >| GUTREV       |                                *

*   | GTRIG  |.. >|    .............. .                                *

*   ........ .    |    .............. .                                *

*                 |.. >|                                               *

*   Digitisations |                                                    *

*                 |    | GUDIGI       |                                *

*                 |    ................                                *

*                 |    ................                                *

*                 |    | Output event |                                *

*                 ... >| GUOUT        |                                *

*                      ................                                *

*   The partition initialized  in GTRIGI is cleared.   The space used  *

*  by the current event may be used by the next one.                   *

*                                                                      *

*                    The banks JRUNG and JHEAD                         *

*                    -------------------------                         *

*                                                                      *

*  Run bank JRUNG:  1 user link, 30 data words                         *

*  LQ(JRUNG-1)   user link                                             *

*   IQ(JRUNG+1) IDRUN     Run number                                   *

*    ""  +2/10)           Reserved for user applications               *

*          +11) creation date for 'INIT' data structures               *

*          +12) creation time for 'INIT' data structures               *

*          +13) creation data for 'KINE'                               *

*          +14) creation time for 'KINE'                               *

*          +15) creation data for 'HITS'                               *

*          +16) creation time for 'HITS'                               *

*          +17) creation data for 'DIGI'                               *

*          +18) creation time for 'DIGI'                               *

*          +19) Random number seed 1                                   *

*          +20) Random number seed 2                                   *

*          +21) GEANT version number when 'INIT' created               *

*          +22) ZEBRA version number when 'INIT' created               *

*          +23) GEANT version number when 'KINE' created               *

*          +24) ZEBRA version number when 'KINE' created               *

*          +25) GEANT version number when 'HITS' created               *

*          +26) ZEBRA version number when 'HITS' created               *

*          +27) GEANT version number when 'DIGI' created               *

*          +28) ZEBRA version number when 'DIGI' created               *

*  Header bank JHEAD: 1 user link, NHEAD(=10) data words               *

*   IQ(JHEAD+1) IDRUN     Run number                                   *

*       ""  +2) IDEVT     Event number                                 *

*       ""  +3) NRNDM(1)  Random number seed 1 at beginning of event   *

*       ""  +4) NRNDM(2)          "     "                              *

*     "" +5/10)           Reserved for user applications               *

*                                                                      *

************************************************************************

*                                                                      *

*                                                                      *

*        Example of User Termination and related routines              *

*        ------------------------------------------------              *

*                                                                      *

*     SUBROUTINE UGLAST                                                *

*  C                                                                   *

*  +SEQ,GCLIST                                                         *

*  C                                                                   *

*  C      Call standard GEANT termination routine                      *

*     CALL GLAST                                                       *

*  C                                                                   *

*  C      Close HIGZ files                                             *

*     CALL IGEND                                                       *

*  C                                                                   *

*  C      Close I/O buffers                                            *

*     IF(NGET.EQ.0.AND.NSAVE.EQ.0) GO TO 5                             *

*     CALL GCLOSE(0,IER)                                               *

*  C                                                                   *

*  C      Print histograms                                             *

*   5 CALL HISTDO                                                      *

*  C                                                                   *

*     END                                                              *

*                                                                      *

*                                                                      *

************************************************************************

#endif